| CAV | CIRCUIT | FUNCTION |
| --- | --- | --- |
| C1 | C13 18DB/RD | A/C COMPRESSOR CLUTCH RELAY CONTROL |
| C2 | - | - |
| C3 | K900 18PK/WT | AUTOMATIC SHUT DOWN RELAY CONTROL |
| C4 | V36 18TN/RD | SPEED CONTROL VACUUM SOLENOID CONTROL |
| C5 | V35 18LG/RD | SPEED CONTROL VENT SOLENOID CONTROL |
| C6 | G68 18BR/YL | OVERDRIVE OFF LAMP DRIVER |
| C7 | - | - |
| C8 | - | - |
| C9 | - | - |
| C10 | J95 18DG/RD | VAPOR CANISTER SOLENOID DRIVER |
| C11 | V32 18YL/RD | SPEED CONTROL ON/OFF SWITCH SENSE |
| C12 | A142 18DG/OR | AUTOMATIC SHUT DOWN RELAY OUTPUT |
| C13 | T9 18OR | OVERDRIVE OFF SWITCH SENSE |
| C14 | J96 18VT/RD | VAPOR CANISTER PUMP SWITCH DRIVER |
| C15 | T222 18RD/YL | BATTERY TEMPERATURE SENSE SIGNAL |
| C16 | - | - |
| C17 | - | - |
| C18 | - | - |
| C19 | K81 18DB | FUEL PUMP RELAY CONTROL |
| C20 | K52 18PK/BK | EVAPORATIVE EMISSION SOLENOID CONTROL |
| C21 | - | - |
| C22 | C3 18DB/BK | A/C PRESSURE SWITCH SENSE |
| C23 | - | - |
| C24 | L53 18BR | STOP LAMP SWITCH SENSE |
| C25 | K72 18DG/VT | VOLTAGE REGULATOR SIGNAL |
| C26 | G40 18LB/BK | LOW FUEL SENSE |
| C27 | D83 18BK/PK | SCI RECEIVE |
| C28 | D2 18WT/BK | CCD BUS(-) |
| C29 | D84 18BK/WT | SCI TRANSMIT |
| C30 | D1 18VT/BR | CCD BUS (+) |
| C31 | - | - |
| C32 | K95 18PK | SPEED CONTROL SWITCH SIGNAL |