Pin PinName Description

1    +5V 5 volts DC

2     data not connected / float

3     data not connected / float

X    GND connected to pin 4 (GND) with 17.3K ohm resistor

4     GND ground

pin X not connected - data cable, nuvi connects to PC as removable drive, navigation NOT possible.
pin X & 4 short-circuited - USB supply 500mA, possible navigation,
pin X & 4 connected through 18kOhm resistor - Garmin supply 1A, possible navigation.
pin X & 4 connected through 22kOhm resistor - Garmin supply 500mA, possible navigation.