Product picture

|  |  | Related products |  |  |  | High-quality products |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Solar charger E.GO! Master

Article No.: 512172 -  62

| Type | e.GO! Master |
| --- | --- |
| Solar module | 2.2 Wp / poly-Si |
| NiMH/Cd energy storage | 4.8-9.6 NiMH/Cd |
| Output power | 1.3 / 2.5 / 4.0 W (continuous) |
| Output voltage | 4.0 - 14 V (stabilised) |
| Dim. mm | 160 x 90 x 36 |
| Weight | 400 g (with 4 AA batteries) |

Price from 126,60 EUR

Top of Form

piece(s)

Bottom of Form

reasonable graduating prices. Unit in ST; Price in EUR; Savings in EUR. 1; 133,26; --. 5; 126,60; 5% = 6,66.

· Charger and battery maintenance unit for small electrical appliances, independent of external power sources 

· Intelligent charging electronics. 


e.GO! combines functions so far found only in advanced mains powered equipment, with the advantages of a solar powered charger and battery maintenance unit. e.GO! features modern solar cells, patented electronics and charging methods and is powered by the built-in high-grade solar cells. e.GO! can charge 2 or 4 AA batteries and small electronic equipment such as mobile phones, PDAs, digital cameras, etc. The energy from the solar cells is stored in the batteries and serves as a buffer so the e.GO! can be used like a conventional charger: from the mains or 12 V connection it can charge round cells and the connected equipment. The output power can be selected for the external equipment, between 1.3 and 4.0 W. The output voltage is set by the adapter cable and ranges from 4 to 14 V. The operating and charge status are indicated by LEDs. The batteries are optimally charged by an intelligent charging process with temperature monitoring and deep discharge protection, supervised by an integrated microprocessor. Additional optional solar modules can be connected externally to improve solar performance. The two solar modules can be separated from the housing so the electronics/battery unit can be used with one, two or no solar modules and you can upgrade from the Fun to the Master model. e.GO! is splashproof and intended for outdoor use. 
Dim. 160 x 85 x 33 mm, weight (w/o batteries) 200 g Fun/300 g Master and Professional. 
Package content: Nokia charger cable as master cable, with adapters for Siemens, Samsung and SonyEricsson, world wide power capability (100 - 240 V), Professional kit also includes: PDA cable, protection, 1 LiIon battery, more powerful mains unit, monocrystalline solar module .