# Coil Springs

#### All springs are interchangeable with the exception of the 110 rears. Those are bigger in diameter, so those are proprietary to 110's.

| PART NUMBER | LOAD LBS | RATE LBS/INCH | RATE KGS/M | COLOUR CODE | FREE LENGTH MM | FREE LENGTH INCH | NO OF COILS | APPLICATION Right HandDrive VEHICLES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| .572315 | 716.00 | 133.00 | 2375.10 | BLUE | 391.16 | 15.4 | 7.18 | RANGE ROVER AND DISCO V8 STD FRONT |
| ANR1975 | 831.12 | 149.89 | 2676.61 | BLUE-RED-PINK |  |  |  | DICOVERY TDI RH FRONT VIN KA TO MA |
| ANR1976 | 922.62 | 149.88 | 2676.43 | BLUE-YELL-PINK |  |  |  | DISCOVERY TDI LH FRONT VIN KA TO MA |
| ANR1977 | 753.11 | 207.84 | 3711.43 | RED-GREY |  |  |  | DISCO REAR VINKA038878 LAO66328 USE ANR3058 |
| ANR3058 | 753.11 | 207.84 | 3711.43 |  |  |  |  | DISCO REA VIN LA066329-LA081991 |
| ANR3477 | 754.94 | 207.84 | 3711.43 | BROWN-GREEN |  |  |  | DISCO REA VIN MA081991-MA119533 |
| ANR3519 | 1069.87 | 149.03 | 2661.25 | RED-BROWN |  |  |  | RANGE ROVER LHD DIESEL ONLY |
| ANR3520 | 852.92 | 150.12 | 2680.71 | YELLOW-BROWN |  |  |  | RANGE ROVER CLASSIC REAR FROM MA647645 |
| ANR3539 | 1007.14 | 182.72 | 3262.86 | WHITE |  |  |  | MILITARY USE ONLY NO SPECS |
| ANR3540 | 2646.00 | 451.09 | 8055.18 | GREEN-PINK |  |  |  | MILITARY USE ONLY NO SPECS |
| ANR3650 | 522.88 | 165.02 | 2946.79 | RED-ORANGE |  |  |  | MILITARY USE ONLY NO SPECS |
| ANR4350 | 831.12 | 149.89 | 2676.61 | 2 BROWN |  |  |  | DISCO FRONT LH VIN MA116461 ON |
| ANR4351 | 822.82 | 149.89 | 2676.61 | 2 GREY |  |  |  | DISCO FRONT RH VIN MA116461 ON |
| ANR4352 | 754.91 | 165.88 | 2962.14 | 2 ORANGE |  |  |  | DISCO REAR OBSOLETE USE 2XANR3477 |
| ANR4491 | 988.88 | 285.50 | 5098.21 | BROWN-GREY |  |  |  | MILITARY USE ONLY NO SPECS |
| NRC2119 | 863.00 | 150.00 | 2678.70 | GREEN | 409.70 | 16.13 | 7.63 | DISCO TDI FRONT TO HA475972 RR Carb HD front STD rear |
| NRC4234 | 978.84 | 170.04 | 3035.86 | GREEN-YELLOW | 411.48 | 16.20 | 7.00 | RANGE ROVER RH FRONT LHD ONLY |
| NRC4304 | 1106.06 | 170.04 | 3035.00 | RED-WHITE | 450.00 | 17.71 | 7.00 | RANGE ROVER REAR HD |
| NRC4305 | 1069.87 | 149.03 | 2678.70 | RED-YELLOW | 436.40 | 17.18 | 7.65 | RANGE ROVER DIESEL LH FRONT LHD ONLY |
| NRC4306 | 855.85 | 133.04 | 2375.20 | BLUE-WHITE | 417.57 | 16.44 | 7.55 | RANGE ROVER PETROL LH FRONT LHD ONLY |
| NRC6388 | 881.25 | 179.87 | 3211.96 | 2 BLUE | 420.00 | 16.53 | 7.50 | 110 REAR LEVELLED SUSPENSION DRIVER |
| NRC6389 | 643.97 | 330.04 | 5893.57 | 2 RED | 407.00 | 16.02 | 8.00 | 110 REAR NON LEVELLED DRIVER HD |
| NRC6904 | 643.97 | 330.04 | 5893.57 | RED-GREEN | 395.00 | 15.55 | 8.00 | 110 REAR PASSENGER NON LEVELLED HD |
| NRC7000 | 881.28 | 179.87 | 3211.96 | GREN-WHITE | 40.00 | 15.75 | 7.50 | 110 REAR DRIVER LEVELLED SUSPENSION |
| NRC8044 | 269.77 | 154.74 | 2763.21 | 2 WHITE | 384.30 | 15.15 | 8.90 | 110 FRONT DRIVER STD |
| NRC8045 | 171.08 | 154.74 | 2763.21 | 2 YELLOW | 368.10 | 14.49 | 8.90 | 110 FRONT PASSENGER STD |
| NRC8113 | 449.82 | 178.20 | 3182.10 | PINK-PURPLE | 418.36 | 16.47 | 8.75 | RANGE ROVER REAR EFI PROGRESSIVE |
| NRC9446 | 819.98 | 175.07 | 3126.25 | BLUE-GREEN | 385.70 | 15.19 | 7.75 | 90 FRONT DRIVER |
| NRC9447 | 819.98 | 175.07 | 3126.25 | BLUE-YELLOW | 373.02 | 14.69 | 7.75 | 90 FRONT PASSENGER |
| NRC9448 | 585.00 | 225.09 | 4019.46 | BLUE-RED | 388.62 | 15.30 | 8.00 | 110FRONT DRIVER HD/130 FRONT DRIVER/90 REAR DRIVER |
| NRC9449 | 585.00 | 225.09 | 4019.46 | YELLOW-WHITE | 376.00 | 14.80 | 8.00 | 110 FRONT PASSENGER HD/130 FRONT PASS/90 REAR PASS |
| NRC9462 | 631.97 | 225.09 | 4019.46 | GREEN-YELL-RED | 387.57 | 15.26 | 9.00 | 90 REAR DRIVER HD |
| NRC9463 | 531.97 | 127.33 | 2273.75 | GREEN-YELL-WHIT | 376.27 | 14.81 | 9.00 | 90 REAR PASSENGER HD |
| NTC3285 | 427.14 | 178.20 | 3182.10 | GREEN-PINK | 461.67 | 18.18 | 8.75 | RANGE ROVER DIESEL REAR LH / LHD DIESEL ONLY |
| NTC8476 | 1034.57 | 149.89 | 2676.61 | WHITE-PINK-BLUE |  |  |  | DISCOVERY DIESEL LH FRONT LHD ONLY |

As usual: No warranty etc.