<<<<Back

Land Rover Coil Springs

Old Man Emu Spring Information 
Les Richmond Automotive
Land Rover Genuine Spring Information 
Bearmach Springs 
Other Spring Manufacturers on the web

If you have any better information or don't agree, please feel free to e-mail me at:
red90 ampersand shaw.ca  “Spam blocking, replace ampersand with @ and no spaces”

Old Man EMU Coil Springs for Land Rovers

From a variety of sources 
http://www.arb.com.au/ 
Please note that some OME information is suspect, so use with caution.  If you have a set of springs loose, please measure the free length, # of coils and bar size and then e-mail me the data.  Thanks...

| Part # | Vehicle(s) | Axle | Application | Length (DS) | Length (PS) | Rate | Bar | Coils |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front |  |  |  | inches | inches | lb/inch | mm |  |
| 760 | Disco, RR (V8) | Front | Light Duty - up to 20 kg extra weight | 15.1 | 14.6 | 180-250 | 16 |  |
| 761 | All V8 | Front | Medium Duty - 20 to 50 kg extra weight | 15.7 | 15.4 | 200 | 16 | 9.1 |
| 751 | All V8 | Front | Heavy Duty - 50 to 110 kg extra weight | 15.3 | 14.9 | 230 | 16 |  |
| 776 | D2 | Front | Light Duty - up to 20 kg extra weight | 16.2 | 15.4 | 160 |  |  |
| 777 | D2 | Front | Medium Duty - 20 to 50 kg extra weight | 15.6 | 14.8 | 180 |  |  |
| 779 | D2 | Front | Heavy Duty - 50 to 110 kg extra weight | 16.2 | 15.2 | 180 |  |  |
| Rear |  |  |  |  |  |  |  |  |
| 752 | D110 | Rear | Light Duty | 17.3 | 17.3 | 270 | 17 |  |
| 764** | Disco, RR, D90 | Rear | Medium Duty | 16.9 | 16.5 | 240 |  | 9.5 |
| 781 | Disco, RR, D90, D2 | Rear | Medium/Heavy Duty | 16.7 | 16.3 | 290 |  |  |
| 762 | Disco, RR, D90, D2 | Rear | Heavy Duty - 100 kg constant load | 16.5 | 16.1 | 300-340 | 18 | 9.5 |
| 763 | Disco, RR, D90, D2 | Rear | Very Heavy Duty - 200 kg constant load | 16.5 | 16.1 | 360 | 18 |  |
| 755 | D110 | Rear | Heavy Duty - 200 kg constant load | 17.1 | 17.1 | 280?? | 19 |  |
| 754 | D110 | Rear | Very heavy Duty - 300 kg constant load | 17.5 | 17.1 | 420 | 21 |  |
| Old |  |  |  |  |  |  |  |  |
| 757 | All V8 | Front | Medium Duty, Replaced by 761 | 15.9 | 15.6 | 180-250 | 16 |  |
| 758 | Disco, RR, D90 | Rear | Medium Duty, Replaced by 764 | 16.9 | 16.5 | 200 | 17 |  |
| 759 | Disco, RR, D90 | Rear | Heavy Duty, Replaced by 762 | 18.1 | 17.7 | 260-300 | 17 |  |

* RR - classic RR only 
** Values modified from OME data and based on actual measurements.

Les Richmond Automotive (Rangie Spares) Springs for Land Rovers

Data supplied by Les Richmond Automotive P/L
41 Wood Street, Thomastown, Victoria, Australia, 3074
Phone 61 3 9464 4094

http://www.lrautomotive.com.au/home.html

Ratings are based on use on classic Range Rovers with load levelers unless noted otherwise

| Code | Duty | Axle | Length | Rate |
| --- | --- | --- | --- | --- |
|  |  |  | inches | lb/inch |
| Yellow | Light | Front | 16.50 | 138 |
| Blue | Light | Front | 15.50 | 160 |
| Green | Heavy | Front | 16.10 | 160 |
| Pink | Extra Heavy | Front | 15.90 | 180 |
| Red | XX Heavy | Front | 16.50 | 180 |
| Purple/Yellow | XXX Heavy | Front | 15.50 | 220 |
| Red | Light | Rear | 16.50 | 180 |
| Orange | Heavy | Rear | 17.20 | 180 |
| Purple | Extra Heavy | Rear | 17.00 | 220 |
| Purple/Orange | Xfi Heavy | Rear | 18.00 | 220 |
| White | XX Heavy | Rear | 16.80 | 270 |
| White/Green | XXX Heavy | Rear | 17.80 | 270 |
| None | 110 rears | Rear | 16.80 | 250 |

Land Rover Coil Springs (Genuine OEM)

compiled by Jim Allen on Four Wheeler and from Land Rover Workshop Manuals

| Part # | Vehicle(s) | Axle | Length | Rate | Coils | Colour Code |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | inches | lb/inch |  |  |
| NRC4306 | Disco V8 '90-'96 RR '87-'91 | Front left (LHD) | 16.44 | 133 | 7.55 | blue/white |
| 572315 | Disco V8 '90-'96 RR '87-'91 | Front right (LHD) Front (RHD) | 15.4 | 133 | 7.18 | blue |
| NRC2119 | Disco TDI to HA475972 RR Carb front HD | Front (RHD) Front right (LHD) | 16.13 | 150 | 7.63 | green |
| NRC 8476 | Disco Diesel LHD | Front right | 16.55 | 150 | 7 | white/blue/pink |
| NRC4305 | Disco 200TDI and RR diesel | Front left (LHD) | 17.18 | 150 | 7.65 | red/yellow |
| NRC4234 | RR LHD | Front right | 16.20 | 170 | 7.00 | green/yellow |
| NRC9446 | D90, Disco HD | Front drivers side | 15.19 | 175 | 7.75 | blue/green |
| NRC9447 | D90, Disco HD | Front passenger side | 14.80 | 175 | 7.75 | blue/yellow |
| NRC8477 | Disco w/ winch kit | Front | 18.18 | 178 | 8.75 | green/blue/yellow |
| NRC8044 | D110 | Front left | 15.13 | 155/215^ | 9^ | white/white |
| NRC8045 | D110 | Front right | 14.49 | 155/215^ | 9^ | yellow/yellow |
| NRC9448 | D110 HD, D130 | Front drivers side | 15.31 | 225 | 8 | blue/red |
| NRC9449 | D110 HD, D130 | Front passenger side | 14.80 | 225 | 8 | yellow/white |
| Rear Springs |  |  |  |  |  |  |
| NRC2119 | RR | Rear * | 16.13 | 150 | 7.63 | green |
| NRC4305 | RR | Rear * | 17.18 | 150 | 7.65 | red/yellow |
| NRC4304 | RR HD | Rear | 17.71 | 170 | 7 | red/white |
| NTC3285 | RR '87-'91 | Rear left * | 17.13 | 180 | 10.21 | green/pink |
| NRC8113 | RR '87-'91 | Rear right * | 16.47 | 180 | 8.75 | pink/purple |
| NRC6388 | D110 with load leveler | Rear passenger side* | 16.53 | 180 | 7.5 | blue/blue |
| NRC7000 | D110 with load leveler | Rear drivers side* | 15.75 | 180 | 7.5 | green/white |
| NTC5527 | Disco V8 '90-'93 | Rear (RHD) | 16.06 | 210 | 9.2 | red/pink |
| NRC8044 | Disco | Rear | 15.13 | 155/215^ | 9^ | white/white |
| NRC8045 | Disco | Rear | 14.49 | 155/215^ | 9^ | yellow/yellow |
| NRC9462 | D90 HD | Rear drivers side | 15.25 | 210/240^ | 9.2 | green/yellow/red |
| NRC9463 | D90 HD | Rear passenger side | 14.81 | 210/240^ | 9.2 | green/yellow/white |
| NRC9448 | D90 | Rear | 15.31 | 225 | 8 | blue/red |
| NRC9449 | D90 | Rear | 14.80 | 225 | 8 | yellow/white |
| RKB101111 | D110 (VIN XZ159807 up) | Rear | 15.0 | 270 to 420 | 8.8 | brown/putple |
| NRC6904** | 110 HD | Rear passenger side | 15.55 | 330 | 8 | red/green |
| NRC6389** | 110 HD | Rear driver side | 16.02 | 330 | 8 | red/red |
| RRC3266 | 130 Inner Helper Springs | Rear | ?? | 140 | 9 | ?? |

* Asterix denotes vehicle with load leveler

** 110 and 130 rear springs are a different coil diameter than other LR springs (nominally 7" OD versus 6")
- Specs are for V8 models only U.N.O. 
- RR - classic RR only

^ Data changed from published value to match those calculated from measurements.  I am using normal spring theory, but a lower than normal modulus of rigidity (70 GPa versus 78 GPa) for the OEM springs as this seems to better match measured springs and published data.

Land Rover Coil Springs (Genuine OEM Incomplete Data)

Here are some newer springs listed without full data.  If anyone has one out, please send me the free lengths.

| Part # | Vehicle(s) | Rate | Colour Code |
| --- | --- | --- | --- |
|  |  | lb/inch |  |
| ANR1975 | Disco TDI Right front, VIN KA to MA | 150 | blue/red/pink |
| ANR1976 | Disco TDI Right front, VIN KA to MA | 150 | blue/yellow/pink |
| ANR1977 | Disco rear VINKA038878 to LA066328 | 208 | red/grey |
| ANR3058 | Disco rea VINLA066329 to LA081991 | 208 | Unknown |
| ANR3477 | Disco Rear MA081991 to MA119533 | 208 | brown/green |
| ANR3519 | RR LHD Diesel rear | 150 | red/brown |
| ANR3520 | RR rear from MA647645 | 150 | yellow/brown |
| ANR3539 | Military | 183 | White |
| ANR3540 | Military | 451 | Green/pink |
| ANR3650 | Military | 165 | Red/Orange |
| ANR4350 | Disco front LH from MA116461 | 150 | brown/brown |
| ANR4351 | Disco front RH from MA116461 | 150 | grey/grey |
| ANR4352 | Disco Rear - Obsolete - Use ANR3477 | 166 | 2 Orange |
| ANR4491 | Military | 286 | Brown/Grey |

Bearmach Springs for Land Rovers

from Bearmach
http://www.bearmach.com

| Part # | Axle | Application | Length | Coil Size | Rate |
| --- | --- | --- | --- | --- | --- |
|  |  |  | inches | mm | lb/inch |
| BA2104 | Front | RR/Disco | 15.5 | 16 | 195 |
| BA2101 | Front | 90/110 | 15.0 | 17 | 255 |
| BA2105 | Rear | Disco | 17.2 | 16 | 194 |
| BA2107 | Rear | RR | 16.2 | 16 | 196 |
| BA2103 | Rear | 90/RR/Disco | 16.2 | 17 | 240 |
| BA2102 | Rear | 90/RR/Disco | 17.5 | 18 | 295 |
| BA2100 | Rear | 110/130 | 16.1 | 22 | 510 |

Other Spring Manufacturer Links

King Springs 
Lovell Springs 
Polyair 
Pedders 
Rocky Mountain Parabolic Springs

Updated 2005/11/26