| Engine |  |
| --- | --- |
| Type | 449cc, 4-Stroke Single, Liquid Cooled w/ Fan, Titanium 5-Valve DOHC |
| Bore x Stroke | 95mm x 63.4mm |
| Compression Ratio | 11.6:1 |
| Fuel Delivery | Yamaha Fuel Injection (YFI) , 42mm |
| Ignition | TCI (Digital) |
| Starting System | Electric |
| Transmission | 5-Speed Manual Clutch |
| Drive Train | 2WD; Sealed O-Ring Chain, Eccentric Adjustment |
| Chassis |  |
| Suspension/Front | Independent Double Wishbone, 9.8”. Kashima Coated Piggy Back Shocks, High/Low Speed Compression, Rebound and Threaded Preload Adjustment. |
| Suspension/Rear | Cast Aluminum Swing Arm, 11.0”. Piggy Back High/Low Speed Compression, Rebound and Threaded Preload Adjustment. |
| Brakes/Front | Dual Ventilated Hydraulic Disc , Twin Piston |
| Brakes/Rear | Wave Style Ventilated Hydraulic Disc, Twin Piston |
| Tires/Front | AT21x7-10 Radial |
| Tires/Rear | AT20x10-9 Radial |
| Dimensions |  |
| L x W x H | 70.7 x 48.8 x 41.9 in |
| Seat Height | 31.9 in |
| Wheelbase | 50.0 in |
| Ground Clearance | 4.5 in |
| Fuel Capacity | 2.6 gal |
| Wet Weight | 405 lb |
| Other |  |
| Lighting | Dual 30W Krypton multi-reflector Headlights and 3.9/0.5W LED Brake light |
| Warranty | 6 Month (Limited Factory Warranty) |