LUCAS 10AS ALARM - Pin Outs

Details of the pin usage for the ECU connector(s). Grey Connector. 1; Interior lights – inputs. 2; Not used. 3; Volumetric signal - input. 4; DDS serial link - output. 5; Drivers door – input. 6; Passengers still - input. 7; Drivers still – input. 8; Door key switch - input. 9; Bonnet. 10; Ignition – input. 11; Malfunction indicator lamp - input. 12; Ground – input. 13; Not used. 14; Active low immobiliser - output. 15; Active high immobiliser - output. 16; Passenger's doors / tailgate - input. 17; Serial comms. 18; Not used. 19; Not used. 20; Volumetric power - output. 21 - 23; Not used. 24; Serial ground – input. 25; Battery voltage – input. 26; Aerial – input. Green Connector. 1; Hazard right - output. 2; CDL lock - output. 3; CDL unlock - output. 4; Security LED - output. 5; Sounder drive - output. 6; Hazards left - output. 7; Transponder drive - output. 8; Hazard power input. 9; EIWL - output. 10; Crank output. 11; Ground - input. 12; Transponder power - output.