| Computer Trouble Codes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Computer Trouble Codes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1990-1993: W201 with 2.3L M102 engine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Flashes |  |  | Fault |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | No System Malfunction |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  | Throttle Valve Swttch |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | Coolant Temperature Sensor |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  | Airflow Sensor Position Indicator |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  | Oxygen Sensor |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | NotUsed |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  | Td Signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  | Altitude Correction Capsule |  |  |  |  |  |  |  |  |  |  |  |
|  | 9 |  | Electro-Hydraulic Actuator (EHA) |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | Throttle Valve Switch and/or Idle Speed Contact |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 11 |  | NotUsed |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 |  | EGR Temperature Sensor |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1991-??: W124, and W202 with 2.8L & 3.2L M104 engines [W129 & W140?] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Flashes |  |  | Fault | - |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | No faults in system |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  | Oxygen Sensor Inoperative |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | Lambda Control Inoperative |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  | Air Injection inoperative |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  | Exhaust Gas Recirculation (EGR) inoperative |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | Idle Speed Control Inoperative |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  | Ignition System Failure |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  | Coolant Temperature Sensor - Open Or Short Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 9 |  | lntake Air Temperature Sensor - Open Or Short Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | Voltage At Air Mass Sensor Too High Or Low |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 11 |  | TN (RPM) Signal Defective |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 |  | Oxygen Sensor Heater Open Or Short Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | Camshaft Position Sensor Signal From - EZLJAKR Ignition Control Unit Defective |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 |  | Intake Manifold Pressure At Start Too Low |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | Full Throttle Information Defective |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 16 |  | Idle Speed Information Defective |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17 |  | Controller Area Network (CAN) Data Exchange - Malfunction Between Control Units |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | Adjustable Camshaft Timing Solenoid - Open Or Short Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19 |  | Fuel Injectors- Open Or Short Circuit, or Emission Control System Adaptation at Limit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 |  | Speed Signal Missing |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21 |  | Purge Switchover Valve - Open Or Short Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 22 |  | Camshaft Position Sensor Signal Defective |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23 |  | Intake Manifold Pressure With Engine Running Too Low |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 |  | Starter Ring Gear Segments Defective |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | Knock Sensors Defective |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 26 |  | Upshift Delay Switch over Valve - Open Or Short Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27 |  | Coolant Temperature Sensor Deviation Between Sensor Circuit No. 1 and Sensor Circuit No. 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | Coolant Temperature Sensor (Coolant Temperature Change Monitor) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1991-1993: W124, W126 & W201 with 2.6L & 3.0L M103 engines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Flashes |  |  | Fault |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | No faults in system |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 |  | Throttle Valve Switch (Full Throttle Contact) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | Coolant Temperature Sensor |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  | Airflow Sensor Potentiometer |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 |  | Oxygen Sensor |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | Not assigned |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7 |  | TNA (Engine RPM) Signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  | Altitude Pressure Signal From EZL Ignition Control Unit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 9 |  | Current To Electro-hydraulic Actuator |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | Throttle Valve switch (Idle Contact) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 11 |  | Air Injection System |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 |  | AbsoIute Pressure Valves From EZL Ignition Control Unit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | Intake Air Temperature Signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 |  | Road Speed Signal At CIS-E Control Unit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | Not assigned |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 16 |  | Exhaust Gas Recirculation (EGR) |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17 |  | Oxygen Sensor Signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | Current To Idle Speed Air Valve |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19 |  | Not assigned |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 |  | Not assigned |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21 |  | Not assigned |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 22 |  | Oxygen Sensor Heating Current |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23 |  | Short Circuit To Positive In Regeneration Switch over Valve Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 |  | Not assigned |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | Short Circuit To Positive in Start Valve Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 26 |  | Short Circuit To Positive In Shift Point Retard Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27 |  | Data Exchange Fault Between CIS-E Control Unit and EZL ignition Control Unit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | Loose Contact In Coolant Temperature Sensor Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | Difference In Coolant Temperatures Between CIS-E Control Unit and EZL Ignition Control Unit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | Not assigned |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21 |  | Loose Contact In Intake Air Temperature Sensor Circuit |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 32 |  | Not assigned |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 33 |  | Not assigned |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 34 |  | Faulty Coolant Temperature Sensor Signal from EZL Ignition Control Unit |  |  |  |  |  |  |  |  |  |  |  |  |  |