### FLUID CAPACITIES

#### FUEL TANK

| All | 78 L (20.5 gal.) |
| --- | --- |

#### ENGINE OIL W/FILTER CHANGE

| 4.0L | 5.7 L (6.0 qts.) |
| --- | --- |
| 4.7L | 5.7 L (6.0 qts.) |

#### COOLING SYSTEM

CAUTION: Vehicle may be equipped with long life coolant. Extended life coolant is identified by an orange color. Normal coolant is identified by a greenish color. It is recommended that the two types of coolant not be mixed.

| 4.0L | 12.3 L (13.0 qts.)* |
| --- | --- |
| 4.7L | 12.3 L (13.0 qts.)* |

*Includes 2.2 L (2.3 qts.) for coolant recovery bottle.

#### AUTOMATIC TRANSMISSION

Dry fill capacity. *

| 42RE | 9.1-9.5L (19-20 pts.) |
| --- | --- |
| 45RFE | 13.33 L (28.0 pts.) |

* Depending on type and size of internal cooler, length and inside diameter of cooler lines, or use of an auxiliary cooler, these figures may vary. Refer to Transmission for proper fluid fill procedure.

#### TRANSFER CASE

| 242 NVG | 1.4 L (3.0 pts.) |
| --- | --- |
| 247 NVG | 1.1 L (2.5 pts.) |

#### FRONT AXLE

| 186 FBI | 1.18 L (2.5 pts.) |
| --- | --- |
| 186FBI‡ | 1.19L (2.51 pts.) |

‡If the vehicle is equipped with VARI-LOK, include 0.07L (0.15 pts.) of friction modifier.

#### REAR AXLE

| 198 RBI* | 1.6 L (3.5 pts.) |
| --- | --- |
| 198RBI ‡ | 1.78L (3.76 pts.) |
| 226 RBA* | 2.24 L (4.75 pts.) |
| 226RBA ‡‡ | 2.25L (4.75 pts.) |

* If the vehicle is equipped with TRAC-LOK, include 0.11 L (0.25 pts.) of friction modifier.

‡ If the vehicle is equipped with a VARI-LOK, include 0.09L (0.19 pts.) of friction modifier.

‡‡ If the vehicle is equipped with a VARI-LOK, include 0.12L (0.25 pts.) of friction modifier.

NOTE: Vehicles with trailer tow, must use a synthetic lubricant. Refer toDifferential and Driveline for service procedures.

#### POWER STEERING

Power steering fluid capacities are dependent on engine/chassis options as well as steering gear/cooler options. Depending on type and size of internal cooler, length and inside diameter of cooler lines, or use of an auxiliary cooler, these capacities may vary. Refer to Section 19 of the MDS2 for proper fill and bleed procedures.