SS 143HF Installation Guide Click Image above for High Resolution Diagram TOYOTA HILUX 3.0L TURBO DIESEL (1KZ-TE). Fitment to: Left hand side Fitting time: Approximately 120 minutes Special Tools: - 92mm Holesaw - Air Hacksaw - Step Drill - Sensor Safe Silicone Sealant. 1; Remove: - LHS front wheel arch liner. - Air entry duct. - Boom bottle from wheel arch.. 2; Remove air entry duct from air cleaner to inner guard. Using a hacksaw carefully cut and remove 90 degree bend from duct (See drawing for detail). Refit air entry duct.. 3; Tape template to upper rear corner of left hand guard. With a felt pen mark all holes. Drill a 4mm pilot hole for each hole position. Drill main hole to 92mm using the holesaw. Drill the remaining 6 holes to 16mm using the step drill. Using the air hacksaw enlarge the main hole as per template. De-burr and paint all holes.. 4; Fasten upper mounting bracket and studs to snorkel. Position snorkel on guard and check for clearance to door when opening. If necessary position snorkel forward to achieve required clearance. Mark 3 holes on “A” pillar. Remove snorkel and drill holes to 8mm. Paint holes and insert plastic body clips to “A” pillar.. 5; Install snorkel to guard and fasten using necessary hardware.. 6; Install snorkel connecting hose and fasten hose clamps. Ensure there is clearance between hose and guard.. 7; Refit wheel arch liner.. WARNING: IT IS THE INSTALLERS RESPONSIBILITY TO ENSURE ALL CONNECTIONS ARE WATER TIGHT.. ITEM; IMAGE; DESCRIPTION; QTY; PART NO.. 1; BODY - SNORKEL; 1; 315-133-200. 2; 3 1/2" AIR RAM ASSEMBLY; 1; 000-135-650. 3; BRACKET - UPPER MOUNTING; 1; 315-032-400. 4; BOLT - HEX 6.0mm x 1.0 x 14mm; 3; 000-002-100. 5; SCREW - S/T 4.2mm x 13mm; 3; 000-951. 6; BODY CLIP - PLASTIC; 3; 000-960. 7; HOSE - AIR ENTRY; 1; 315-024-000. 8; STUD - S/S 8mm x 1.25mm x 25mm O/L; 6; 000-110. 9; WASHER - BODY 8mm x 30mm; 6; 000-311. 10; NUT - NYLOC 8mm x 1.25mm; 6; 000-205. 11; CLAMP - HOSE 70/90mm; 2; 000-908. 12; TEMPLATE; 1; 315-017-200. 13; HOSE CLAMP - SIZE 56 (BLACK); 1; 000-914/B.