PROFESSIONAL FM TRANSCEIVER

Frequency range: 65-108MHz (Rx only)

136-174MHz(RX/TX)	175-220MHz(RX/TX)

230-250MHz(RX/TX)	330-350MHz(RX/TX)

350-400MHz(RX/TX)	400-520MHz(RX/TX)

• Non-standard CTCSS(60-260HZ)and DCS(D000N/I-D777N/I)

• DTMF coding and decoding

• CTCSS DCS automatic scanning

• Various colors for display background (optional)

• Multiple side keys with customized functions Radio

• Stopwatch

• Suitable for working frequencies of different nations and regions

• Special digital sub-audio function

• Frequency sweeping function • Huge color screen

• GPS function (model optional)

USER'S MANUAL

Preface

Thank you for buying the Abbree products.

This product offers latest design, enhanced features, solid performances and easy accessibility. We believe you will be pleased with the high quality and reliable features for all your communication needs. This manual includes functional descriptions and step-by-step instructions. It also includes troubleshooting guides. If the body is damaged due to incorrect operation, please read the safety information manual carefully before use.

CONTACT US:

Email address: info@abbree.com

Official website: www.abbree.com

■Highlight Function

- GPS function

- Fast scan frequency and pairing

- All calls, group calls and selective calls

- DTMF encoding/Decoding

- Up to 128 named memory channels.

- 128-64 full dot matrix wide viewing angle LCD screen, Six colors background

- Manual input frequency and programming

- Twin band simultaneous receiving (U-U,U-V,V-U,V-V)

■Main Features

-Frequency range: 65-108MHz (Rx only)

136-174MHz(RX/TX)

175-220MHz(RX/TX)

230-250MHz(RX/TX)

330-350MHz(RX/TX)

350-400MHz(RX/TX)

400-520MHz(RX/TX)

(Within 136-174MHz and 400-470MHz, the power more higher and the communication distance more longer)

-Friendly man-machine interface, easier to operate

-Dual-band, dual-display, dual-watch, Dual-band handheld transceiver

-CTCSS/DCS, DTMF signalling

-Voice Operated Transmit (VOX)

-Emergency alarm and ANI identification through DTMF

-Equipped with the relay pilot and scramble function

-SOS emergency alert function

-Priority scan, priority channel setting

-CTCSS/DCS Scan, channel scan

-PC programming (frequency reading and writing frequency) will be password protected -Broadcast FM radio receiver 65-108MHz

-Frequency step, selectable between2.5K | 5.0K | 6.25K | 10.0K | 12.5K | 20.0K | 25.0K | 50.0K

-High Capacity Lithium-Ion battery

-Programmable repeater offset -LED flashlight

-USB Micro charging, battery life is more convenient

Table of Contents

Contents

Chapter 1. Getting started	4

1.1. Packing List	4

1.2. Assembly	4

1.2.1Antenna	4

1.2.2. Belt clip	4

1.2.3. Battery	4

1.2.4. Charging and battery maintenance	5

1.3. Getting to know your radio	6

1.3.1. Radio Controls and Indicators	6

1.3.2. Reading the display	7

1.3.3 Status LED	7

1.3.4. The maim keys	7

Chapter 2. Basic Operations	9

2.1. Power and volume	9

2.2. Making a call	9

2.3. Channel selection	9

2.4. Frequency (VFO) mode	9

2.5Channel (MR) mode	10

Chapter 3. Advanced Operations	10

3.1. Working the menu system	10

3.2. Scanning	11

3.3. Fast scan frequency and pairing	12

3.4 Dual Watch	13

3.5. DCS, CTCSS and Tone-burst	13

3.6. DTMF	14

3.7. DTMF call	15

3.8. FM Radio	16

3.9. VOX (Voice activated transmit)	16

3.10. Manual programming	17

! WARNING	18

Chapter 4. Product Safety Guide	18

Important Safety Information	18

FCC Statement	19

Appendix A. - Troubleshooting	20

Appendix B. Technical Specifications	21

Appendix C. - Menu definitions	22

Appendix D. - CTCSS Table	24

Appendix E. - DCS Table	25

# Chapter 1. Getting started

## 1.1. Packing List

Please unpack carefully and check that you have received the following items. If any item is missing or damaged, please contact your dealer.

| Item | Quantity (PCS) | Item | Quantity (PCS) |
| --- | --- | --- | --- |
| Radio | 1 | Antenna | 1 |
| Belt Clip | 1 | Wrist Strap | 1 |
| Charging Cradle | 1 | Documentation Kit | 1 |
| AC-DC Charging Adapter | 1 | Lithium Ion Poly Battery | 1 |

! The frequency band is marked on the label of antenna; if it is unavailable there; see the label on the radio or contact our company.

## 1.2. Assembly

Before the radio is ready for use we need to attach the antenna and battery pack, as well as charge the battery.

### 1.2.1Antenna

This transceiver is fitted with a Male SMA connector. To mount your antenna (Female SMA Connector), align the two connectors and turn clockwise until it stops.

•Do not over-tighten your antenna to avoid damage to the connectors.

- When installing the antenna, don't grip it by the top. Grip by the base and turn.

- Do not hold the antenna with your hand or wrap the outside of it to avoid bad operation of the transceiver.

- Never transmit without an antenna.

### 1.2.2. Belt clip

At the back of the radio there are two parallel screws mounted above the battery, remove these and thread them through the holes on the belt clip as you screw them back into the radio body.

! Do not use any form of glue to fix the screws on the battery clip. The solvents in the glue may cause damage to the battery casing.

### 1.2.3. Battery

Before attaching or removing the battery make sure your radio is turned off by turning the power/volume knob all the way counter-clockwise.

■Installation

Make sure the battery is aligned in parallel with the radio body with the lower edge of the battery about 1-2cm below the edge of the radio.

! Once aligned with the guide-rails, slide the battery upward until you hear a click as the battery locks in place.

■Removal

To remove the battery, press the battery release above the battery pack, as you slide the battery downward.

### 1.2.4. Charging and battery maintenance

■Charging

! Battery should be fully charged before initial use. Optimum battery efficiency will be achieved after the three full battery charge and discharge cycles.

Follow these steps to hook up and use the charger:

- Plug the DC connector of the power adaptor into the charger base.

- Plug the AC connector of the power adaptor into a main AC wall outlet.

- Place the radio in the charging slot on the charger.

- Make sure the radio is making contact with the charger. When the red LED comes on steady, your radio is charging.

- The radio is fully charged once the charger's green status LED goes steady. Please remove the radio at that time to avoid over-charging your battery.

To determine the charging status, check the light-emitting diode (LED) indicator on the charger according to the following table:

| Charging Indicator | Charging Status |
| --- | --- |
| Glows red | The battery is charging. |
| Glows green | The battery is fully charged. |
| Flashes red rapidly | The battery fails to be charged. |

! The charger and battery are fitted with matching notches so that you can charge your battery on its own! Practical if you have two batteries. That way you can charge one battery while still using your radio.

Radio should be turned OFF during charge cycle.

Press and hold the [0] key to display the battery voltage.

Lithium Ion Battery Warning

This equipment contains a rechargeable Lithium Ion battery. The rechargeable Lithium Ion battery contained in this equipment may explode if disposed of in a fire.

- Never attempt to disassemble your battery pack.

- Do not short-circuit the battery.

- Use only batteries approved by the original manufacturer.

- Do not charge the rechargeable battery used in this equipment in any charger other than the one specified in the owner’s manual. Using another charger may damage the battery or cause the battery to explode.

- The battery for your radio comes uncharged from the factory; please let it charge for at least four to five hours before you start using your radio.

! Li-lon batteries must be recycled or disposed of properly.

Avoid exposing the battery (whether attached to the radio or not) to direct sunlight, heated cars, or temperatures below-4° F (-20° C) or above +140° F (+60° C). Exposing the chemical contained within the battery pack to temperatures above +140° F (+60° C) may cause the battery to rupture, fail, or reduce performance.

In case of exposure to the cell contents, wash the affected area thoroughly, and seek medical attention.

■Storage

Partially charge your battery before long-term storage in order to prevent damage from over-discharge. While lead acid must always be kept at full charge during storage, this radio uses a lithium-based battery and should be stored at around a 40 percent charge. This level minimizes age-related capacity loss while keeping the battery in operating condition and allowing self-discharge.

To avoid severe capacity degradation of your battery while in long-term storage, please cycle the battery at least every six (6) months.

Store your batteries in a cool and dry place, never above normal room temperatures.

## 1.3. Getting to know your radio

### 1.3.1. Radio Controls and Indicators

Figure 1.3.1. Radio Controls and Indicators

| 1. Antenna | 2. Channel selector knob | 3. Power/Volume knob | 4. Battery release latch |
| --- | --- | --- | --- |
| 5. Accessory jack | 6. Battery pack | 7. Battery contacts | 8. PTT key |
| 9. MENU function key | 10. Side key2 (PF2) | 11. Side кеуЗ (PF3) | 12. A / B select key |
| 13. Microphone | 14. Numeric keypad | 15. Status LED | 16. Dual band LCD |
| 17. Navigation key | 18. EXIT key | 19. VFO/MR mode key | 20.Speaker |

! You can request your dealer to program the following keys as shortcuts to radio features: PF2 key, and PF3 key. Please consult your dealer for assignable radio features.

## 1.3.2. Reading the display

The LCD display shows different information depending on what you are doing. This sample screen shows some of this information. The following table shows all the possible icons and what they mean.

Figurel.3.2. Reading the display

| Icon | Description | Icon | Description |
| --- | --- | --- | --- |
|  | Squelch Open/ Close Indicator | power | power is set to Hi ( 8W), Mi( 1W) |
|  | Battery level indicator | D | Dual watch enabled |
|  | VOX enabled | N | Narrowband enabled |
|  | The keys are locked. | + | TX will be shifted higher in frequency than RX |
|  | Indicates active band or channel | - | TX will be shifted lower in frequency than RX |
| DCS | DCS enabled | CT | CTCSS enabled |
| 10/2 | Current channel number | R | Reverse function enabled |
|  | This icon appears when DTMFST is set to DT-ST/ANI-ST/ DT+ANI |  |  |

### 1.3.3 Status LED

The status LED has a very simple and traditional design. When you receive a signal it turns green, when you transmit it turns red, and it's off in standby.

| LED Indicator | Radio Status |
| --- | --- |
| Glows red | Transmitting. |
| Glows green | Receiving |

### 1.3.4. The maim keys

■Side key 2 -SK2 (Flashlight and alarm)

Monitor

Press [SK2] momentarily to turn on the LED flashlight. Another momentary press will flash the LED. Another momentary press turns the flashlight off.

Press and hold [SK2] to activate the alarm function. Press (a short press) again to turn it off.

■Side key 3-SK3 (Broadcast FM and Monitor)

Press [SK3] momentarily to start the broadcast FM receiver. Another momentary press turns the broadcast FM receiver off. If a signal is received on the active frequency or channel while you are listening to the broadcast FM, the receiver will open squelch to that frequency (as if scanning) and remain there until the signal goes away; it will then switch back to broadcast FM.

Press and hold [SK3] to monitor the signal. This will open up the squelch so you can listen to the unfiltered signal.

■V/M (VFO / MR) - mode key

Press [V/M] switches between Frequency (VFO) Mode and Memory (MR) mode. Memory mode is sometimes also referred to as Channel mode.

To save frequencies to channel memory you must be in Frequency (VFO) mode.

■A / B select key

The [A/B] key switches between A (upper) and B (lower) displays. The frequency or channel on the selected display becomes the active listening and transmit frequency or channel.

To save frequencies to channel memory you must be on the A display.

■Numeric keypad

The radio comes standard with a full numeric keypad.

The numeric keys have their secondary function printed on them (in reality it's rather menu short-cuts, more on that in Chapter 3.1, working the menu system).

The [*scan] and [#lock] keys on the other hand have actual secondary functions, scan and keypad lock respectively.

Press and hold the [0] key to display the battery voltage

! The charger and battery are fitted with matching notches so that you can charge your battery on its own! Practical if you have two batteries. That way you can charge one battery while still using your radio.

Radio should be turned OFF during charge cycle.

■Pound [#lock] Key

When listening to broadcast FM a momentary press [#lock] will start the scanning. Scanning in broadcast FM will stop as soon as an active station is found, regardless of scanner resume method.

The radio features a keypad lock that locks out all keys except for the three side keys.

To enable or disable the keypad lock, press and hold the [#lock] key for about two seconds.

You can also enable so that the radio automatically locks the keypad after ten seconds from the menu, see Chapter 3.1, working the menu system (menu 23).

■Star [*scan]/scan Key

To enable the scanner, press and hold the [*scan] key for about two seconds. See Chapter 3.2, scanning for details.

■Menu and function keys

-The [MENU] key, used to enter the menu and confirm menu options.

-The [UP] and [DOWN] keys are used to navigate through the menu as well as select channels and step up or down in frequency (depending on operating mode).

-The [EXIT] key is used to exit menus and cancel menu options.

For a more in-depth explanation on how to work the menu see Chapter 3.1, working the menu system.

# Chapter 2. Basic Operations

## 2.1. Power and volume

! Before we turn the power on, make sure you have attached the battery and antenna.

■Turning the unit on

To turn the unit on, simply rotate the volume/power knob clockwise until you hear a "click". If your radio powers on correctly there should be an audible double beep after about one second and the display will show a message or flash the LCD depending on settings for about one second. Then it will display a frequency or channel. If the Voice prompt is enabled, the voice will announce "frequency mode" or "channel mode".

Figure 2.1.1.Frequency mode	Figure	2.1.2.Channel mode

■Turning the unit off

Turn the volume/power knob counter-clock wise all the way until you hear a "click". The unit is now off.

■Adjusting the volume

To turn up the volume, turn the volume/power knob clock-wise. To turn the volume down, turn the volume/power knob counter-clock-wise. Be careful not to turn it too far, as you may inadvertently turn your radio off.

## 2.2. Making a call

! Press the [А/В] key to switch the main channel to the other channel if there is 2 channels shown on the display. Press the [V/M] key to switch between VFO and channel display.

- Call channel mode: After selecting a channel, hold down the [PTT] key to initiate a call to the current channel. Speak into the microphone with normal tone. Initiate a call, the red LED is on.

- Frequency mode call: Press the [V/M] key to switch to the frequency mode, the frequency range allowed entering, press the [PTT] key, a call to the current channel. Speak into the microphone with normal tone. Initiate a call, the red LED is on.

- Receive a call: When you release the [PTT] key, you can answer it without any action.

When receiving a call, the green LED is on.

! To ensure the best reception volume, keep the distance between the microphone and the mouth at the time of transmission from 2.5 cm to 5 cm.

## 2.3. Channel selection

There are two modes of operation: Frequency (VFO) mode, and Channel or Memory (MR) mode.

For everyday use, Channel (MR) mode is going to be a whole lot more practical than Frequency (VFO) mode. However, Frequency (VFO) mode is very handy for experimentation out in the field. Frequency (VFO) mode is also used for programming channels into memory.

In Channel (MR) mode you can navigate up and down the channel by using the [UP] and [DOWN] keys.

Ultimately which mode you end up using will depend entirely on your use case.

## 2.4. Frequency (VFO) mode

In Frequency (VFO) mode you can navigate up and down the band by using the [UP] and [DOWN] keys. Each press will increment or decrement your frequency according to the frequency step you've set your transceiver to.

You can also input frequencies directly on your numeric keypad with kilohertz accuracy.

The following example assumes the use of a 12.5 kHz frequency step.

Example 2.4.1. Entering the frequency 462.625 MHz on display A

- Use the [V/M] key to switch to Frequency (VFO) mode.

- Press [A/B] until the ▲ icon appears next to the upper display.

- Enter [4] [6][2][6][2][5] on the numeric keypad.

! WARNING

Just because you can program in a channel does not mean you're automatically authorized to use that frequency. Transmitting on frequencies you're not authorized to operate on is illegal, and in most jurisdictions a serious offence. If you get caught transmitting without a license you can and will get fined, and in worst case sent to jail. However, it is legal in most jurisdictions to listen. Contact your local regulatory body for further information on what laws, rules and regulations apply to your area.

## 2.5Channel (MR) mode

The use of Channel (MR) mode is dependent on actually having programmed in some channels to use.

Once you have channels programmed and ready, you can use the [UP] and [DOWN] keys to navigate between channels.

# Chapter 3. Advanced Operations

Part two covers the more advanced operations, such as setup of repeater offset and programming via computer link.

3.1. Working the menu system

3.2. Scanning

3.3. Fast scan frequency and pairing

3.4. Dual watch

3.5. CTCSS, DCS and Tone-burst

3.6. DTMF

3.7. DTMF call

3.8. FM Radio

3.9. VOX

3.10. Manual programming.

## 3.1. Working the menu system

For a complete reference on available menu items and parameters, see Appendix C, Menu definitions.

■Basic use

Procedure 3.1.1. Using the menu with arrow keys

1. Press the [MENU] key to enter the menu.

2. Use the [UP] and [DOWN] keys to navigate between menu items.

3. Once you find the desired menu item, press [MENU] again to select that menu item.

4. Use the [UP] and [DOWN] keys to select the desired parameter.

5. When you've selected the parameter you want to set for a given menu item;

- To confirm your selection, press [MENU] and it will save your setting and bring you back to the main menu.

- To cancel your changes, press [EXIT] and it will reset that menu item and bring you out of the menu entirely.

6. To exit out of the menu at any time, press the [EXIT] key.

■Using short-cuts

As you may have noticed if you looked at Appendix C, Menu definitions, every menu item has a numerical value associated with it. These numbers can be used for direct access of any given menu item.

The parameters also have a number associated with them; see Appendix C, Menu definitions for details.

Procedure 3.1.2. Using the menu with short-cuts

- Press the [MENU] key to enter the menu.

- Use the numerical keypad to enter the number of the menu item.

- To enter the menu item, press the [MENU] key.

- For entering the desired parameter you have two options:

- Use the arrow keys as we did in the previous section; or

- Use the numerical keypad to enter the numerical short-cut code.

- And just as in the previous section;

- To confirm your selection, press [MENU] and it will save your setting and bring you back to the main menu.

- To cancel your changes, press [EXIT] and it will reset that menu item and bring you out of the menu entirely.

- To exit out of the menu at any time, press the [EXIT] key.

## 3.2. Scanning

The radio features a built in scanner for the VHF and UHF bands. When in Frequency (VFO) mode it will scan in steps according to your set frequency step. In Channel (MR) mode it will scan your channels.

Dual Watch is inhibited while scanning

! To enable the scanner, press and hold the [*scan] key for about two seconds. Press [#lock] key to exit scanning mode.

■Scanning modes

The scanner is configurable to one of three ways of operation: Time, carrier or search, each of which is explained in further details in their respective section below.

Procedure 3.2.1 Setting scanner mode

- Press the [MENU] key to enter the menu.

- Enter [1][8] on your numeric keypad to come to scanner mode.

- Press the [MENU] key to select.

- Use the [UP] and [DOWN] keys to select scanning mode.

- Press the [MENU] key to confirm and save.

- Press the [EXIT] key to exit the menu.

-Time operation

In Time Operation (TO) mode, the scanner stops when it detects a signal, and after a factory present time out, it resumes scanning.

-Carrier operation

In Carrier Operation (CO) mode, the scanner stops when it detects a signal, and after a factory present time with no signal it resumes scanning.

-Search operation

In Search Operation (SE) mode, the scanner stops when it detects a signal. To resume scanning you must press and hold the [*scan] key again.

• in the CPS Software Path: - Edit - Optional Features - Scan Mode, it can set three scan modes, and program the desired scan mode and write it to the radio.

Tone Scanning

Scanning for DCS and CTCSS Codes/ Tones

Scanning for a DCS code or CTCSS tone can be done while Frequency Mode (VFO) or Channel Mode (MR) is selected. Only when VFO mode is selected, can the detected code / tone be saved to menu 11/12.

! DCS code and CTCSS tone scanning mode can be accessed with or without a signal being present. The scanning process itself only occurs while a signal is being received.

Not all repeaters requiring a DCS code or CTCSS tone for access will transmit one back. In that case, the transmitter of a station that can access the repeater would need to be scanned. In other words: this would be done by listening to stations on the repeater's input frequency.

Scanning for a DCS tone

- Press [MENU] key to enter the menu.

- Enter [l][l]on your numeric keypad to come to Menu 11: RX DCS.

- Press the [MENU] key to select. Insure you have a tone activated (and it is not off).

- Press the [*scan] to begin DCS scanning.

Scanning for CTCSS Tone

- Press [MENU] key to enter the menu.

- Enter [1][2] on your numeric keypad to come to Menu 12: RX CTCS.

- Press the [MENU] key to select. Insure you have a tone activated (and it is not off).

- Press the [*scan] to begin CTCSS scanning.

Procedure 3.2.2 CPS programming Channel scanning

1. Channels are added to the scan group via CPS programming software.

In the channel list, the Scan Add option is set to ON, the channel is added to the scan group.

Figure 3.2.1Add channel scan

•in the CPS Software Path: - "Edit" - "Optional Features - Scan Mode", it can set three scan modes, and program the desired scan mode and write it to the radio.

## 3.3. Fast scan frequency and pairing

This unit can be regarded as a receiver to quickly implement frequency monitoring, pairing, and call-back functions.

- In standby mode, press [MENU] [*scan] to enter the scan receiving status, and the received frequency and sub tone code will be displayed on the screen.

- Press [UP]/[DOWN] to switch the UHF/VHF band in the scan.

- When the received frequency and subsonic code are relatively stable, pressing the [MENU] key of the unit will automatically store the received frequency and sub tone code and turn on the speaker.

- Press and hold the [PTT] key of this unit to realize callback.

! The distance between the transmitter and the receiver is not less than 50 cm.

## 3.4 Dual Watch

The radio features Dual Watch functionality (single receiver) with the ability to lock the transmit frequency to one of the two channels it monitors.

You can both have one receiver in your radio and flip-flop between two frequencies at a fixed interval (known as Dual Watch), or you can equip a radio with two receivers (known as Dual Receive or Dual VFO).

Procedure 3.4. Enabling or disabling Dual Watch mode

- Press [MENU] key to enter the menu.

- Enter [7] on the numeric keypad to get to Dual Watch.

- Press [MENU] to select.

- Use the [UP] and [DOWN] keys to enable or disable.

- Press the [MENU] key to confirm.

- Press the [EXIT] key to exit the menu.

## 3.5. DCS, CTCSS and Tone-burst

Some times when you're working with larger groups of people using the same channel, things can get very crowded, very fast. To minimize this problem, several methods of blocking out unwanted transmissions on your frequency have developed. In general, there are two forms of selective calling in two-way radio systems: Group calling, and individual calling.

Group calling is a one-to-many form of communication. Every radio in your working group is configured the same way and any radio will make contact with every other radio in the group. Individual calling, sometimes also known as paging, is a one-to-one form of communication. Every radio is programmed with a unique ID code. And only by sending out a matching code can you get that radio to open up to your transmissions.

The radio features three different ways of group calling:

- DCS

- CTCSS

- Tone-burst (1750Hz)

The radio does not feature any form of individual calling.

Using these features does NOT mean that others won't be able to listen in on your transmissions.

They only provide a method to filter out unwanted incoming transmissions. Any communications made while using these features will still be heard by anyone not employing filtering options of their own.

Also, you cannot change the DCS or CTCSS settings while in memory (MR) mode.

CTCSS and 1750Hz tone-burst are also popular methods among amateur radio operators to open up repeaters.

■DCS

DCS is set with menu 11 RX DCS and menu 13 TX DCS.

For a complete list of available DCS codes, see Appendix E, "DCS Codes".

Procedure 3.5.1 DCS setup how-to

- Press the [MENU] key to enter the menu.

- Enter [1][1] on the numeric keypad to get to receiver DCS.

- Press [MENU] to select.

- Enter desired DCS code on the numeric keypad.

- Press [MENU] to confirm and save.

- Enter [UP] on the numeric keypad to go to transmitter DCS.

- Press [MENU] to select.

- Enter desired DCS code on the numeric keypad. Make sure it's the same code as that you entered for receiver DCS.

- Press [MENU] to confirm and save.

- Press [EXIT] to exit the menu system.

For more information see the section called "11 RX DCS - Receiver DCS" and the section called "13 TX DCS - Transmitter DCS" in Appendix C, Menu definitions.

■CTCSS

CTCSS is set with menu 12 RX CTCS and menu 14 TX CTCS.

For a complete list of available CTCSS codes and corresponding sub-tone frequencies, see Appendix D, CTCSS Table.

Procedure 3.5.2 CTCSS setup how-to

- Press the [MENU] key to enter the menu.

- Enter [1][2] on the numeric keypad to get to receiver CTCSS (RX CTC).

- Press [MENU] to select.

- Enter desired CTCSS sub-tone frequency in hertz on the numeric keypad.

- Press [MENU] to confirm and save.

- Enter [UP] on the numeric keypad to go to transmitter CTCSS (TX CTC).

- Press [MENU] to select.

- Enter desired CTCSS sub-tone frequency in hertz on the numeric keypad. Make sure it's the same frequency as that you entered for receiver CTCSS.

- Press [MENU] to confirm and save.

- Press [EXIT] to exit the menu system.

For more information see the section called "12 RX CTCS - Receiver CTCSS" and the section called "14 TX CTCS - Transmitter CTCSS" in Appendix C, Menu definitions.

■ 1000Hz, 1450Hz, 1750Hz, 2100Hz Tone-burst

In the transmitting state, simultaneously press and hold the [PTT] and the [SK2] to send the selected tone-burst.

Procedure 3.5.3 Tone-burst setup how-to

- Press the [MENU] key to enter the menu.

- Enter [1][6] on the numeric keypad to get to tone-burst.

- Press [MENU] to select.

- Use the [UP] and [DOWN] keys to select between 1000Hz, 1450Hz, 1750Hz and 2100Hz.

- Press [MENU] to confirm and save.

- Press [EXIT] to exit the menu system.

## 3.6. DTMF

DTMF is an in-band signaling method using dual sinusoidal signals for any given code. Originally developed for telephony systems, it has proved a very versatile tool in many other areas.

In two-way radio systems, DTMF is most commonly used for automation systems and remote control. A common example would be in amateur radio repeaters where some repeaters are activated by sending out a DTMF sequence (usually a simple single-digit sequence).

|  | 1209Hz | 1336Hz | 1477Hz | 1633Hz |
| --- | --- | --- | --- | --- |
| 697Hz | 1 | 2 | 3 | A |
| 770Hz | 4 | 5 | 6 | B |
| 852Hz | 7 | 8 | 9 | C |
| 941Hz | * | 0 | # | D |

The radio has a full implementation of DTMF, including the А, В, C and D codes.

! The numerical keys, as well as the [*scan] and [#lock] keys correspond to the matching DTMF codes. The А, В, C and D codes are located in the [MENU],[UP],[DOWN]and[EXIT] keys respectively.

## 3.7. DTMF call

CPS programmed

The CPS programming settings of the DTMF call function include programming of important parameters such as DTMF code group, local ID code, and group call code.

Before using the DTMF selective call function, make the following settings through the CPS programming software:

Procedure 3.7.1 CPS programming setup how-to

- DTMF encoding / decoding parameters Edit

In the CPS software - "Edit" - "DTMF - DTMF Encoder / Decoder Edit", the important parameters such as DTMF Encoder group, ANI code, group code are programmed.

Figure 3.7.1 DTMF Encoder/ Decoder Edit

- Set the signalling code, PTT-ID mode

Method 1: PC settings

In the CPS software - "Edit" - "Channel Information", it can signal the channel selection, PTT-ID
mode selection, and write it to the radio.

Method 2: Radio menu settings

- Press the [MENU] key to enter the menu.

- Enter [1][7] on the numeric keypad to get to S-CODE.

- Press [MENU] to select.

- Use the [UP] and [DOWN] keys to select 1-15.

- Press [MENU] to confirm and save.

- Press [EXIT] to exit the menu system.

## 3.8. FM Radio

The FM radio function of the radios defaults to long press the [SK3] key.

Procedure 3.8.1 Activating FM radio

In standby, press [SK3] to activate the FM Radio function, while it shows the FM frequencies on the display.

Procedure 3.8.2 Searching FM stations

In the FM radio mode, press [#scan] to get it searching. When searching the correct station, it stops.

Procedure 3.8.3 Exit from the FM radio

Please press [SK3] to exit out from the FM radio mode.

! When working on the FM frequencies, the current frequency and channel will be standby and it will be temporarily switched to two-way communication once getting the receiving signals, and then automatically get back to FM radio after the signal disappears. Press [PTT] to transmit, and still gets back to FM radio after 5 seconds.

## 3.9. VOX (Voice activated transmit)

Enable the VOX, you can speak into the microphone to start transmitting instead of pressing the [PTT] key. A total of 10 levels are provided.

- Press the [MENU] key to enter the menu.

- Enter [4] on the numerical keypad, to enter the Vox Level.

- Press [MENU] to select.

- Press the [UP] or [DOWN] key to select 1 to 10.

-The larger the value, the higher the sensitivity.

- Press [MENU] to confirm and save.

- Press [EXIT] to exit the menu.

- Repeat the above operation and select OFF to turn off the voice-activated transmission function.

## 3.10. Manual programming

Memory channels are an easy way to store commonly used frequencies so that they can easily be retrieved at a later date.

The radio features 128 memory channels that each can hold: Receive and transmit frequencies, transmit power, group signalling information, bandwidth, ANI/ PTT-ID settings and a six character alphanumeric identifier or channel name 1.

Frequency Mode vs Channel Mode

Switch between Modes by using the [V/M] key.

These two modes have different functions and are often confused.

Frequency Mode (VFO) - Used for a temporary frequency assignment, such as a test frequency or quick field programming if permitted.

Channel Mode (MR) - Used for selecting preprogrammed channels.

All programming must be initially none in the frequency mode (VFO) only. From there you have the option of assigning the entered data to a specific channel for access in the channel mode.

Call tones, TX/RX tones, squelch, and power settings are adjustable on saved channels in channel mode.

Ex. Programming a Simplex Channel with CTCSS tone

EXAMPLE New memory in Channel 11:

RX/TX = 462.625 MHz TX CTCSS tone 67.0 Procedure 3.10.1 Programming a Simplex Channel with CTCSS tone

- Set radio to VFO Mode by pressing [V/M] Channel number at the right will disappear.

- Press [MENU][2][7][MENU] to come to Menu 27: Delete, Press [1][0][MENU] to deletes prior data in channel (Ex. CH-11).

- Press [EXIT] to exit the menu. Enter RX frequency [4][6][2][6][2][5] on you numeric keypad.

- Press [MENU][1][2][MENU] to come to Menu 12: RX CTCS, press [UP]/[DOWN] to select

- and then press [MENU] to confirm.

- Press [MENU][1][4][MENU] to come to Menu 14: TX CTCSS, press [UP]/[DOWN] to select

- and then press [MENU] to confirm.

- Press [MENU][2][6][MENU] to come to Menu 26: Memory, and then Press [1][0][MENU] to confirm (Ex. CH-11).

- Press [V/M] return to MR Mode. Channel number will re-appear.

Ex: Programming a Channel Repeater Offset with CTCSS Tone

EXAMPLE New memory in Channel 20:

RX = 462.625 MHz TX = 452.625 MHz (This is a (+ 10.00) Offset)

TX CTCSS tone 67.0 RX CTCSS tone 67.0 Procedure 3.10.2 Programming a Channel Repeater Offset with CTCSS Tone

- Set radio to VFO Mode by pressing [V/M] Channel number at the right will disappear.

- Press [MENU] [2] [7] [MENU] to come to Menu 27: Delete, Press [1][9][MENU] to deletes prior data in channel (Ex. CH-20).

- Press [EXIT] to exit the menu. Enter RX frequency [4][6][2][6][2][5] on you numeric keypad.

- Press [MENU] [1][2] [MENU] to come to Menu 12: RX CTCS, press [UP]/[DOWN] to select and then press [MENU] to confirm.

- Press [MENU] [1][4] [MENU] to come to Menu 14: TX CTCSS, press [UP]/[DOWN] to select and then press [MENU] to confirm.

- Press [MENU] [2][5] [MENU] to come to Menu 25: Offset, enter offset frequency [1][0][0][0][0] on you numeric keypad.

- Press [MENU] [2][4][MENU] to come to Menu 24: Direction, press [UP]/[DOWN] to select Plus or Minus offset.

- Press [MENU] [2] [6] [MENU] to come to Menu 26: Memory, and then Press [1][9][MENU] to confirm (Ex 20).

- Press [V/M] return to MR Mode. Channel number will re-appear.

! Certain Amateur Radio repeaters (especially in Europe) use a 1750Hz tone burst to open up the repeater.

If you're still unable to make a connection, contact the person in charge of the radio system with your employer or your local amateur radio club, as the case may be.

If you for some reason want to listen to the repeater's input frequency instead, press [MENU] and [*] momentarily and you'll reverse your transmit and receive frequencies.

Commercial Radio Setup

PLMR users in the United States are mandated to move to 12.5 kHz narrowband communication in the 150-174 MHz VHF and 421-512MHz UHF bands by January 1, 2013. Follow these instructions to set your radio to Narrowband mode: This section is only true for VFO mode. WN is settable on a per channel basis and has to be set prior to storing a channel. Once a channel has been programmed, the channel must be deleted and reprogrammed to change the WN setting.

- Press the [V/M] key to enter frequency mode.

- Press the [MENU] key to enter the menu.

- Enter [5] on the numerical keypad.

- Press [MENU] to select.

- Use the [UP] and [DOWN] keys to select between Wide and Narrow.

- Press [MENU] to confirm and save

- Press [EXIT] to exit the menu.

Amateur Radio Setup

In contrast with Commercial radio operators, who often need very specific requirements to be compatible with a very specific radio implementation, Amateur radio operators tend to need the broadest possible settings in order to be compatible with as many systems as possible. This basically implies turning all the fancy features that you typically might need for a commercial setup off.

In a typical Amateur radio setup the following settings would be recommended:

- Set bandwidth to Wide (menu item 5).

- Turn DCS and CTCSS off (menu items 11 through 14).

- Turn ROGER, DTMF-ST, PTT-ID off (menu item 30, menu items 15 and menu itemsl9).

- Turn off Squelch Tail Elimination (STE) features (menu items 29).

For further information see Appendix C, Menu definitions and Chapter 3.1, working the menu system.

! WARNING

FRS, GMRS, MURS, PMR446

You may be tempted to use FRS, GMRS, MURS (in the USA) or PMR446 (in Europe) frequencies. Do note however that there are restrictions on these bands that make this transceiver illegal for use.

# Chapter 4. Product Safety Guide

Important Safety Information

Before using your radio, please read these general precautions and warnings.

Warning and Notice Statements

To make the most of this radio, it must be used properly.

Please read the installation and operating instructions carefully before using the radio. Special attention must be paid to the WARNING and NOTICE statements in this manual.

! WARNING

Emends identify conditions that could result in personal injury or loss of life.

! notice

Statements identify conditions that could cause damage to the radio or other equipment.

FCC Statement

This device complies with Part 15 of the FCC Rules. Operation is subject to the following two (2) conditions:

- This device may not cause harmful interference, and

- This device must accept any interference received, including interference that may cause undesired operation.

! NOTE

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

! NOTE

Any changes or modifications to this unit not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

Safety Training Information

This radio complies with the following guidelines and standards regarding RF energy and electromagnetic energy levels as well as evaluation of those levels for human exposure:

United States Federal Communications Commission, Code of Federal Regulations; 47 CFR § 1.1307, 1.1310 and 2.1093

- American National Standards Institute (ANSI) / Institute of Electrical and Electronic Engineers (IEEE) C95.1:2005; Canada RSS102 Issue 5 March 2015

Institute of Electrical and Electronic Engineers (IEEE) C95.1:2005 Edition

The following WARNINGS and NOTICE information will make you aware of RF exposure hazards and how to assure you operate the radio within the FCC RF exposure limits established for the radio.

! WARNINGS

To control your exposure and ensure compliance with the occupational/ controlled environmental exposure limits, always adhere to the following procedures.

Guidelines:

-DO NOT remove the RF Exposure Label from the device.

-User awareness instructions should accompany device when transferred to other users.

-DO NOT use this device if the operational requirements described herein are not met.

Operating Instructions:

- DO NOT operate with more than a duty cycle of 5% transmit, 5% receive and 90% standby. The radio is transmitting when the PTT key is pressed and the transmit information shows on the LCD screen.

- Your radio generates electromagnetic RF (radio frequency) energy when it is transmitting.

To ensure that you and those around you are not exposed to excessive amounts of that energy, DO NOT touches the antenna when transmitting. KEEP the radio at least two (2) inches (5 cm) away from yourself and others when transmitting.

- When worn on the body, always place the radio in an approved holder, holster, case, or body harness or by use of the correct clip for this product. Use of non-approved accessories may result in exposure levels which exceed the FCC's occupational/ controlled environmental RF exposure limits.

- DO NOT allow children or anyone unfamiliar with proper procedures to operate the radio without supervision.

- Use of non-approved antennas, batteries, and accessories causes the radio to exceed the FCC RF exposure guidelines.

- Contact your local dealer for the product's optional accessories.

- Changes or modifications to your radio MAY VOID its compliance with FCC (Federal Communications Commission) rules and make it illegal to use.

- Failure to observe any of these warnings may cause you to exceed FCC RF exposure limits or create other dangerous conditions.

EU Regulatory Conformance

As certified by the qualified laboratory, the product is in compliance with the essential requirements and other relevant provisions of the following directives:

- 2014/53/EU

- 2012/19/EU

- 2011/65/EU

- 2006/66/EU

Please note that the above information is applicable to EU countries only.

# Appendix A. - Troubleshooting

| Phenomena | Analysis | Solution |
| --- | --- | --- |
| You cannot turn on the radio. | The battery may be installed improperly. | Remove and reattach the battery. |
|  | The battery power may run out. | Recharge or replace the battery. |
|  | The battery may suffer from poor contact caused by dirty or damaged battery contacts. | Clean the battery contacts or replace the battery. |
| During receiving, the voice is weak or intermittent. | The battery voltage maybe low. | Recharge or replace the battery. |
|  | The volume level may be low. | Increase the volume. |
|  | The antenna maybe loose or maybe installed incorrectly. | Turnoff the radio, and then remove and reattach the antenna. |
|  | The speaker maybe blocked. | Clean the surface of the speaker. |
| You cannot communicate with other group members. | The frequency or signaling type maybe inconsistent with that of other members. | Verify that your TX/RX frequency and signaling type are correct. |
|  | You may be too far away from other members. | Move towards other members. |
| You hear unknown voices or noise. | You may be interrupted by radios using the same frequency. | Change the frequency, or adjust the squelch level. |
|  | The radio in analog mode maybe set with no signaling. | Request your dealer to set signaling for the current channel to avoid interference |
| You are unable to hear anyone because of too much noise and hiss. | You may be too far away from other members. | Move towards other members. |
|  | You may be in an unfavorable position. For example, your communication may be blocked by high buildings or blocked in an underground area. | Move to an open and flat area, restart the radio, and try again. |
|  | It may be the result of external disturbance (such as electromagnetic interference). | Stay away from equipment that may cause interference. |
| The radio keeps transmitting. | VOX may be turned on or the headset is not installed in place | Turn off the VOX function. Check that the headphones are in place. |
| You cannot use the keys. | The keypad may not work temporarily. | Restart the radio. |

! lf the above solutions cannot fix your problems, or you may have some other queries, please contact your dealer for more technical support.

# Appendix B. Technical Specifications

General

Frequency Range 65-108MHz (Rx only)

136-174MHz(RX/TX)

175-220MHz(RX/TX)

230-250MHz(RX/TX)

330-350MHz(RX/TX)

350-400MHz(RX/TX)

400-520MHz(RX/TX)

Memory Channel			128 Groups

Operation Voltage			DC 7.4 V ±10%

Battery Capacity			2800mAH (Li-Ion)

Frequency Stability		±2.5ppm

Operating Temperature		-20°C to +60°C

Mode of Operation		Simplex

Antenna Impedance		50ohm

Transmitter Part

RF Output Power			8W

FM Modulation			11K0F3E@12.5KHz

Spurious Emission			-36dBm < 1GHz	, -30dBm > 1GHz

Adjacent Channel Power		60dB @ 12.5KHz

Transmission current		<1600mA

Receiver Part

Receive Sensitivity			0.25pV (12dB SINAD)

Adjacent Channel Selectivity	>55dB@12.5KHz

Inter Modulation and Rejection	>55dB@12.5KHz

Conducted Spurious Emission	<-57dB@12.5KHz

Rated Audio Power Output	1W @16 ohms

Receive current			S380mA

Rated Audio Distortion		<5%

! All specifications may be modified without prior notice or liability. Thank you.

# Appendix C. - Menu definitions

| Menu № | Name (Full name) | Setting | Description |
| --- | --- | --- | --- |
| 0 | Squelch - Squelch level | [0-9] | Setting the squelch to 0 will open up the squelch entirely. - Squelch silences the receiver when there is no signal. - Setting the squelch to 0 will open up the squelch entirely. |
| 1 | Step - Frequency Step | 2.5K[0] ¦ 5K[1] ¦ 6.25K[2] ¦ 10.00K [3]¦ 12.50K[4]¦ 20.0K[5] ¦ 25.0K[6] ¦ 50.0K[7] | Selects the amount of frequency change in VFO/Frequency mode when scanning or pressing the [up] or [DOWN] keys. |
| 2 | Tx Power- Transmitting Power | High[0]¦ Low[l] ¦ Middle[2] | Selects between High, Middle, and Low transmitter power when in VFO/Frequency mode. Use the minimum transmitter power necessary to carry out the desired communications. |
| 3 | Power save-Battery Save | OFF[0]¦ 0N[1] | Select to activate or deactivate battery saver. Activating this feature will extend battery life, but you may miss the first few syllables before RX opens. -ON: open the function. -OFF: close the function |
| 4 | Vox Level – Voice Operated TX | OFF [0] ¦ 1 ¦ 2 ¦ 3 ¦ 4 ¦ 5 ¦ 6 ¦ 7 ¦ 8 ¦ 9 ¦ 10 [10] | When enabled it is not necessary to push the [PTT] key on the transceiver. Adjust the gain level to an appropriate sensitivity to allow smooth transmission. |
| 5 | Bandwidth -Wideband / Narrowband | Width [0] ¦ Narrow [1] | -Wideband (25 kHz bandwidth) or -Narrowband (12.5 kHz bandwidth). |
| 6 | Backlight- Display Illumination Time | Bright [0] ¦ 1-10 in 1 second steps | Time-out for the LCD backlight, (seconds) |
| 7 | Dual Standby- Dual Watch, Dual Reception | OFF [0] ¦ ON [1] | Monitor [A] and [B] at the same time. The display with the most recent activity ([A] or [B]) becomes the selected display. |
| 8 | Beep Prompt - Keypad Beep | OFF [0] ¦ ON [1] | Allows audible confirmation of a key press. |
| 9 | Voice - Voice Prompt | OFF[0] ¦ ON[l] | Turns the menu action voice prompt on or off. |
| 10 | Tx over time -Transmission Time-out-Tim er | 15[1] - 600[40] in 15 second steps (TIMEOUT-15)/15=[n] | This feature provides a safety switch that limits transmission time to a programmed value. This will promote battery conservation by not allowing you to make excessively long transmissions, and in the event of a stuck PTT switch it can prevent interference to other users as well as battery depletion. |
| 11 | Rx DCS - Receiver DCS | OFF[0] ¦ see DCS Table in Appendix E | Mutes the speaker of the transceiver in the absence of a specific low-level digital signal. If the station you are listening to does not transmit this specific signal, you will not hear anything. |
| 12 | Rx CTCSS – Receiver CTCSS | OFF[0] ¦ see CTCSS Table in Appendix D | Mutes the speaker of the transceiver in the absence of a specific and continuous sub-audible signal. If the station you are listening to does not transmit this specific and continuous signal, you will not hear anything. |
| 13 | Tx DCS - Transmitter DCS | OFF [0] ¦ see DCS Table in Appendix E | Transmits a specific low-level digital signal to unlock the squelch of a distant receiver (usually a repeater). |
| 14 | Tx CTCSS - Transmitter CTCSS | OFF[0] ¦ see CTCSS Table in Appendix D | Transmits a specific and continuous sub-audible signal to unlock the squelch of a distant receiver (usually a repeater). |
| 15 | DTMFST - DTMF Side Tone | OFF [0] ¦ DT-ST[1] ¦ ANI-ST[2]¦ DT+ANI[3] | Determines when DTMF Side Tones can be heard from the transceiver speaker. - OFF: No DTMF Side Tones are heard. - DT-ST: Side Tones are heard only from manually keyed DTMF codes. - ANI-ST: Side Tones are heard only from automatically keyed DTMF codes. - DT+ANI: All DTMF Side Tones are heard. |
| 16 | R-TONE - Tone-burst | 1000Hz[0] ¦ 1450Hz[l] ¦ 1750Hz[2] ¦ 2100Hz[3] | 1000Hz, 1450Hz, 1750Hz, 2100Hz Tone-burst To send out a tone-burst; You simultaneously will holding down the [РП] and [SKI] key. |
| 17 | S-CODE - Signal Code | 1[0] I 2[1] ¦ 3[2] ¦ 4[3] ¦ 5[4] ¦ 6[5] ¦ 7[6] ¦ 8[7] ¦ 9[8] ¦ 10[9] ¦ 11[10] ¦ 12[11] ¦ 13[12] ¦ 14[13] ¦ 15[14] | Selects 1 of 15 DTMF codes. The DTMF codes are programmed with software and are up to 5 digits each. |
| 18 | Scan Mode - Scanner Resume Method | Time[0]¦ Carrier [1] ¦ Search [2] | Scanning Resume Method : Time: Scanning will resume after a fixed time has passed. Carrier: Scanning will resume after the signal disappears. Search: Scanning will not resume. |
| 19 | PTT-ID - When to send the РП-ID | OFF [0] I BOT [1] 1 EOT [2] I BOTH [3] | Determines when to sent PTT-ID. OFF: No ID is sent. BOT: The selected S-CODE is sent at the beginning. EOT: The selected S-CODE is sent at the ending. BOTH: The selected S-CODE is sent at the beginning and ending. |
| 20 | MDF-A - Channel Mode A Display | Name [0] 1 Frequency [1] | [A] Name/Frequency Mode Display Format. Name: Displays the channel name. Frequency: Displays programmed Frequency. Note: Names must be entered using software. |
| 21 | MDF-B - Channel ModeB Display | Name [0] 1 Frequency [1] | [B] Name/Frequency Mode Display Format Name: Displays the channel name. Frequency: Displays programmed Frequency. Note: Names must be entered using software. |
| 22 | Busy Lockout | OFF [0] ¦ ON [1] | Disables the [РП] key on a channel that is already in use. The transceiver will sound a beep tone and will not transmit if the [РП] key is pressed when a channel is already in use. |
| 23 | Key Auto Lock- Automatic Keypad Lock | OFF [0] ¦ ON [1] | When ON, the keypad will be locked if not used in 8 seconds. Pressing the [#] key for 3 seconds will unlock the keypad. |
| 24 | Direction - Frequency Shift Direction | None [0] 1 Plus [1] 1 Minus[2] | Enables access of repeaters in VFO/Frequency Mode. None: TX = RX (simplex). Plus: TX will be shifted higher in frequency than RX. Minus:TX will be shifted lower in frequency than RX. |
| 25 | Offset - Frequency shift amount | Manual input frequency (range 0.001-99.998. eg 05.000) | Specifies the difference between the TX and RX frequencies |
| 26 | Memory - Store a Memory Channel | 001 -128 | This menu is used to either create new or modify existing channels (1 through 128) so that they can be accessed from MR/Channel Mode. |
| 27 | Delete - Delete a memory channel | 001 -128 | This menu is used to delete the programmed information from the specified channel (1 through 128) so that it can either be programmed again or be left empty. |
| 28 | Alarm Mode | On site [0] ¦ Send sound [1] ¦ Send code [2] | On site: Sounds alarm through your radio speaker only. Send sound: Transmits a cycling tone over-the-air. Send code: Transmits ANI code and alarm. |
| 29 | TAIL - Squelch Tail Elimination | OFF [0] ¦ ON [1] | This function is used eliminate squelch tail noise between handholds that are communicating directly (no repeater). Reception of a 55 Hz or 134.4 Hz tone burst mutes the audio long enough to prevent hearing any squelch tail noise. |
| 30 | ROGER - Roger Beep | OFF [0] ¦ ON [1] | Sends an end-of-transmission tone to indicate to other stations that the transmission has ended. |
| 31 | Language | English [0] ¦ Chinese [1] | English or Chinese language prompts (including voice and display) are allowed. |
| 32 | Reset - Restore defaults | VFO [0] ¦ ALL [1] | Resets the radio to factory defaults, with some exceptions. - VFO: Will reset all functional setting and renew factory default parameter of function setting, but not resetting the parameter of channels. - ALL: Will reset all functional setting and the parameter of all channels, and renew factory default of function. |

# Appendix D. - CTCSS Table

CTCSS CHART (Hz)

| Number | Frequency | Number | Frequency | Number | Frequency | Number | Frequency | Number | Frequency |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 67.0 | 2 | 69.3 | 3 | 71.9 | 4 | 74.4 | 5 | 77.0 |
| 6 | 79.7 | 7 | 82.5 | 8 | 85.4 | 9 | 88.5 | 10 | 91.5 |
| 11 | 94.8 | 12 | 97.4 | 13 | 100 | 14 | 103.5 | 15 | 107.2 |
| 16 | 110.9 | 17 | 114.8 | 18 | 118.8 | 19 | 123.0 | 20 | 127.3 |
| 21 | 131.8 | 22 | 136.5 | 23 | 141.3 | 24 | 146.2 | 25 | 151.4 |
| 26 | 156.7 | 27 | 159.8 | 28 | 162.2 | 29 | 165.5 | 30 | 167.9 |
| 31 | 171.3 | 32 | 173.8 | 33 | 177.3 | 34 | 179.9 | 35 | 183.5 |
| 36 | 186.2 | 37 | 189.9 | 38 | 192.8 | 39 | 196.6 | 40 | 199.5 |
| 41 | 203.5 | 42 | 206.5 | 43 | 210.7 | 44 | 218.1 | 45 | 225.7 |
| 46 | 229.1 | 47 | 233.6 | 48 | 241.8 | 49 | 250.3 | 50 | 254.1 |

# Appendix E. - DCS Table

DCS CODE LIST

| Number | Code | Number | Code | Number | Code | Number | Code | Number | Code |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D023N | 2 | D025N | 3 | D026N | 4 | D031N | 5 | D032N |
|  |  |  |  |  | N |  | N |  | N |
| 6 | D036 | 7 | D043 | 8 | D047 | 9 | D051 | 10 | D053 |
|  | N |  | N |  | N |  | N |  | N |
| 11 | D054 | 12 | D065 | 13 | D071 | 14 | D072 | 15 | D073 |
|  | N |  | N |  | N |  | N |  | N |
| 16 | D074 | 17 | D114 | 18 | D115 | 19 | D116 | 20 | D122 |
|  | N |  | N |  | N |  | N |  | N |
| 21 | D125 | 22 | 0131 | 23 | D132 | 24 | D134 | 25 | D143 |
|  | N |  | N |  | N |  | N | * | N |
| 26 | D145 | 27 | D152 | 28 | D155 | 29 | D156 | 30 | D162 |
|  | N |  | N |  | N |  | N |  | N |
| 31 | D165 | 32 | D172 | 33 | D174 | 34 | D205 | 35 | 0212 |
|  | N |  | N |  | N |  | N |  | N |
| 36 | D223 | 37 | D225 | 38 | D226 | 39 | D243 | 40 | D244 |
|  | N |  | N |  | N |  | N |  | N |
| 41 | D245 | 42 | D246 | 43 | D251 | 44 | D252 | 45 | D255 |
|  | N |  | N |  | N |  | N |  | N |
| 46 | D261 | 47 | D263 | 48 | D265 | 49 | D266 | 50 | D271 |
|  | N |  | N |  | N |  | N |  | N |
| 51 | D274 |  | D306 | 53 | D311 | 54 | D315 | 55 | D325 |
|  | N |  | N |  | N |  | N |  | N |
| 56 | D331 | 57 | D332 | 58 | D343 | 59 | D346 | 60 | D351 |
|  | N |  | N |  | N |  | N |  | N |
| 61 | D356 | 62 | D364 | 63 | D365 | 64 | D371 | 65 | D411 |
|  | N |  | N |  | N |  | N |  | N |
| 66 | D412 | 67 | D413 | 68 | D423 | 69 | D431 | 70 | 0432 |
|  | N |  | N |  | N |  | N |  | N |
| 71 | D445 | 72 | D446 | 73 | D452 | 74 | D454 | 75 | D455 |
|  | N |  | N |  | N |  | N |  | N |
| 76 | D462 | 77 | D464 | 78 | D465 | 79 | D466 | 80 | D503 |
|  | N |  | N |  | N |  | N |  | N |
| 81 | D506 | 82 | D516 | 83 | D523 | 84 | 0526 | 85 | D532 |
|  | N |  | N |  | N |  | N |  | N |
| 86 | D546 | 87 | D565 | 88 | D606 | 89 | 0612 | 90 | D624 |
|  | N |  | N |  | N |  | N |  | N |
| 91 | D627 | 92 | D631 | 93 | D632 | 94 | D645 | 95 | D654 |
|  | N |  | N |  | N |  | N |  | N |
| 96 | D662 | 97 | D664 | 98 | D703 | 99 | D712 | 100 | D723 |
|  | N |  | N |  | N |  | N |  | N |
| 101 | D731 | 102 | D732 | 103 | D734 | 104 | 0743 | 105 | D754 |
|  | N |  | N |  | N |  | N |  | N |
| 106 | D023¦ | 107 | D025¦ | 108 | D026¦ | 109 | D031¦ | 110 | D032¦ |
| 111 | D036¦ | 112 | D043¦ | 113 | D047¦ | 114 | D051¦ | 115 | D053¦ |
| 116 | D054¦ | 117 | D065¦ | 118 | D071¦ | 119 | D072¦ | 120 | D073¦ |
| 121 | D074¦ | 122 | D114¦ | 123 | D115¦ | 124 | D116¦ | 125 | D122¦ |
| 126 | D125¦ | 127 | D131¦ | 128 | D132¦ | 129 | D134¦ | 130 | D143¦ |
| 131 | D145¦ | 132 | D152¦ | 133 | D155¦ | 134 | D156¦ | 135 | D162¦ |
| 136 | D165¦ | 137 | D172¦ | 138 | D174¦ | 139 | D205¦ | 140 | D212¦ |
| 141 | D223¦ | 142 | D225¦ | 143 | D226¦ | 144 | D243¦ | 145 | D244¦ |
| 146 | D245¦ | 147 | D246¦ | 148 | D211¦ | 149 | D252¦ | 150 | D255¦ |
| 151 | D261¦ | 152 | D263¦ | 153 | D265¦ | 154 | D266¦ | 155 | D271¦ |
| 156 | D274¦ | 157 | D306¦ | 158 | D311¦ | 159 | D315¦ | 160 | D325¦ |
| 161 | D331¦ | 162 | D332¦ | 163 | D343¦ | 164 | D346¦ | 165 | D351¦ |
| 166 | D361¦ | 167 | D364¦ | 168 | D365¦ | 169 | D371¦ | 170 | D411¦ |
| 171 | D412¦ | 172 | D413¦ | 173 | D423¦ | 174 | D431¦ | 175 | 0432¦ |
| 176 | D445¦ | 177 | D446¦ | 178 | D452¦ | 179 | D454¦ | 180 | D455¦ |
| 181 | D462¦ | 182 | D464¦ | 183 | D465¦ | 184 | D466¦ | 185 | D503¦ |
| 186 | D506¦ | 187 | D516¦ | 188 | D523¦ | 189 | D526¦ | 190 | D532¦ |
| 191 | D546¦ | 192 | D565¦ | 193 | D606¦ | 194 | D612¦ | 195 | D624¦ |
| 196 | D627¦ | 197 | D631¦ | 198 | D632¦ | 199 | D645¦ | 200 | D654¦ |
| 201 | D662¦ | 202 | D664¦ | 203 | D703¦ | 204 | D712¦ | 205 | D723¦ |
| 206 | D731¦ | 207 | D732¦ | 208 | D734¦ | 209 | D743¦ | 210 | D754¦ |

Disclaimer

The accuracy and completeness of the contents are sought in the process of compilation, but we do not bear any responsibility for the possible errors or omissions. With the continuous development of technology, we reserve the right to change the design and specification of the product without notice. This manual may not be reproduced, modified, translated or transmitted in any form without the prior written authorization of the company.

The third party products and contents involved in this manual belong to the third party. Our company does not guarantee its accuracy, effectiveness, timeliness, legality or integrity.

RBRC INFORMATION

As part of our commitment to protect the environment and conserve natural resources, Wanneton voluntarily participates in an RBRC® industry program to collect and recycle used Li-Ion batteries within the US.

Please call 1-800-8-BATTERY for information on Li-Ion battery recycling in your area.

(RBRC® is a registered trademark of the Rechargeable Battery Recycling Corporation.)

Disposal of your Electronic and Electric Equipment

Products with the symbol (crossed-out wheeled bin) cannot be disposed as household waste. Electronic and Electric Equipment should be recycled at a facility capable of handling these items and their waste by products. In EU countries, please contact your focal equipment supplier representative or service centres for information about the waste collection system in your country.

Attention in case of use

This transceiver works on frequencies which are not generally permitted. As for the actual usage, the user has to possess an amateur radio license. Usage is allowed only in the frequency bands which are allocated for amateur radios.

| List of national codes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| AT | BE | BG | CY | CZ | DE |
| DK | ES | EE | FI | FR | GB |
| GR | HR | HU | IE | IT | LT |
| LU | LV | MT | NL | PL | PT |
| RO | SK | SI | SE | СН | IS |
| U | NO |  |  |  |  |

Step frequency - menu 1

Under the standby mode, press button [menu] + [1] the screen will display: STEP

Silencing level - menu 2

Under the standby mode, press button [menu] + [2] the screen will display: SQL

Power saving mode - menu 3

Under the standby mode, press button [menu] + [3] the screen will display: SAVE

Power selection - menu 4

Under the standby mode, press button [menu] + [4] the screen will display: TXP

Transmit prompt tone - menu 5

Under the standby mode, press button [menu] + [5] the screen will display: ROGER

Transmit time-out - menu 6

Under the standby mode, press button [menu] + [6] the screen will display: TOT

Voice-control transmit - menu 7

Under the standby mode, press button [menu] + [7] the screen will display: VOX

Bandwidth selection - menu 8

Under the standby mode, press button [menu] + [8] the screen will display: WN

Voice prompt on-off - menu 9

Under the standby mode, press button [menu] + [9] the screen will display: VOICE

Transmitting warning timing - menu 10

Under the standby mode, press button [menu] + [1] [0] the screen will display: TOA

Press [menu] to enter, press ▲▼ to select the demanded timer and then press [menu] to confirm. Press [exit] to return to standby mode. This interphone has ten levels of transmission alarm timer and every second is a level. Off: turn off the timer.

Attention - When the continuous transmitting time exceeds the time set by the timer, there will be a prompt tone warning the error and the current transmission will be stopped.

Beep switch - menu 11

Under the standby mode, press button [menu] + [1] [1] he screen will display: BEEP

Press [menu] to enter, press ▲▼ to select ON or OFF and then press [menu] to confirm. Press [exit] to return to standby mode.

Menu language- menu 12

Under the standby mode, press button [menu] + [1] [2] he screen will display: LANGUAGE

Press [menu] to enter, press ▲▼ to select the demanded language and then press [menu] to confirm. Press [exit] to return to standby mode. This interphone has two languages: Chinese and English. The prompt tone will change according to the language chosen.

Transmission prohibited during peak hour – menu 13

Under the standby mode, press button [menu] + [1] [3] he screen will display: BCL

Press [menu] to enter, press ▲▼ to select ON or OFF and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - This function is prohibited when the interphone is in transfrequency transition or relay/transmitter mode.

Scanning mode - menu 14

Under the standby mode, press button [menu] + [1] [4] he screen will display: SC-REV

Press [menu] to enter, press ▲▼ to select the demanded scanning mode and then press [menu] to confirm. Press [exit] to return to standby mode. Scanning mode: search, time, and carrier.

TO: When carrier signal is scanned and there is no operation in five seconds, scanning can be continued.

CO: stop scanning when carrier signal is detected and continue to scan three seconds after the carrier signal disappears.

SE: stop scanning on the detection of carrier signal.

Attention - Hold [*scan] or 2 seconds, the scanning mode will be initiated.

Receiving analog sub-audio - menu15

Under the standby mode, press button [menu] + [1] [5] he screen will display: R-CTC

Press [menu] to enter, press ▲▼ to select the demanded sub-audio and then press [menu] to confirm. Press [exit] to return to standby mode. Scanning mode: search, time, and carrier.

There are 50 sets of standard analog sub-audio within the range of 67.0-254.1Hz. Off: turn off.

Transmitting analog sub-audio - menu16

Under the standby mode, press button [menu] + [1] [6] he screen will display: T-CTC

Press [menu] to enter, press ▲▼ to select the demanded sub-audio and then press [menu] to confirm. Press [exit] to return to standby mode. There are 50 sets of standard analog sub-audio within the range of 67.0 - 254.1 Hz. OFF: turn off.

Receiving digital sub-audio - menu 17

Under the standby mode, press button [menu] + [1] [7] he screen will display: R-DCS

Press [menu] to enter, press ▲▼ to select the demanded sub-audio and then press [menu] to confirm. Press [exit] to return to standby mode. There are 105 sets of positive code and 105 sets of negative code within the range of D023N – D754I. OFF: turn off.

Transmitting digital sub-audio - menu 18

Under the standby mode, press button [menu] + [1] [8] he screen will display: T-DCS

Press [menu] to enter, press ▲▼ to select the demanded sub-audio and then press [menu] to confirm. Press [exit] to return to standby mode. There are 105 sets of positive code and 105 sets of negative code within the range of D023N – D754I. OFF: turn off.

Type of digital sub-audio - menu 19

Under the standby mode, press button [menu] + [1] [9] he screen will display: DCS-TYPE

Press [menu] to enter, press ▲▼ to select the demanded parameter type and then press [menu] to confirm. Press [exit] to return to standby mode. This function includes two types of digital sub-audio: NORMAL and SPECIAL.

Definition of side key 2 - menu 20

Under the standby mode, press button [menu] + [2] [0] he screen will display: PF2

Press [menu] to enter, press ▲▼ to select the demanded parameter type and then press [menu] to confirm. Press [exit] to return to standby mode.

The parameter of side key 2 includes: scanning, long lamp, SOS, alarm, radio, monitoring, selected calling and turning off. The selected calling code can be edited by the software we provide.

Definition of side key 3 - menu 21

Under the standby mode, press button [menu] + [2] [1] he screen will display: PF3

Press [menu] to enter, press ▲▼ to select the demanded parameter type and then press [menu] to confirm. Press [exit] to return to standby mode.

The parameter of side key 3 includes: scanning, long lamp, SOS alarm, remote alarm, radio, monitoring, selected calling and turning off. The parameters above can realize the following functions:

1. PF1 for scanning function

Press PF1 under the standby mode to enter scanning mode (scanning mode depends on the setting of menu 14) and the scanning process can be stopped by pressing any key.

2. Selecting long lamp by PF1

Long hold PF1 to initiate the long lamp under the standby mode and press again to turn it off.

3. Selecting SOS-CH alarm by PF1

Press PF1 under the standby mode. Two seconds later, an alarm will blare.

Attention - Each alarm will last for 10 seconds and the interval between two alarms is 5 minutes. Press any key to quit this function.

4. Selecting remote alarm by PF1

An alarm and an alarm code will blare when PF1 is pressed under the standby mode. Press PTT to quit the alarm.

5. Selecting radio by PF1

Under the standby mode, press PF1, the screen will display “76.0 MHz” Then press [*scan] to search channels. The search will stop automatically when radio is found.

Attention - When the radio is working, the current frequency or channel is in the state of standby. It will return to interphone mode upon detection of signal. It returns to radio mode five seconds after the signal disappears. It returns to interphone mode automatically five seconds after pressing the PTT to transmit.

Working mode - menu 22

Under the standby mode, press button [menu] + [2] [2] he screen will display: CH-MDF

Press [menu] to enter, press ▲▼ to select the demanded parameter type and then press [menu] to confirm. Press [exit] to return to standby mode.

The working mode of interphone can be set as VFO frequency mode and MR channel mode which includes four different display modes.

A. Channel number

B. Frequency + channel number

C. Channel name

D. Frequency

Attention - VFO frequency mode and MR channel mode sometimes maybe restricted by a password, which requires correct password to exchange them.

If there is a mode exchanging password, press “[VFO/MR]” the screen will display: CH-MDF

Press function key to confirm after input. The modes will be exchange when the password is correct. The password can only be set by the frequency-writing software we provide. The mode exchanging restriction will be concealed when the password is 000000.

Backlight selection - menu 23

Under the standby mode, press button [menu] + [2] [3] he screen will display: ABR

Press [menu] to enter, press ▲▼ to select parameter that needs to be set and then press [menu] to confirm. Press [exit] to return to standby mode.

Always ON: the backlight is always on.

1S - 20S: set the time that the backlight is turned off regularly.

Always OFF: the backlight is always off (this function is invalid since this interphone has color screen).

Frequency difference selection - menu 24

Under the standby mode, press button [menu] + [2] [4] he screen will display: OFFSET

Press [menu] to enter, press ▲▼ to select the demanded frequency difference and then press [menu] to confirm. Press [exit] to return to standby mode.

The frequency difference is between 0 - 599.995 MHz. The seventh and eighth digit will be automatically identified by the stepping frequency.

Frequency difference direction - menu 25

Under the standby mode, press button [menu] + [2] [5] he screen will display: SFT-D

Press [menu] to enter, press ▲▼ to select the demanded frequency difference direction and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - The frequency difference direction means:

a. Positive direction(+) when the transmitting frequency is higher than receiving frequency

b. Negative direction (-) when the transmitting frequency is lower than the receiving frequency

c. Turning off the frequency difference direction (OFF)

Stopwatch switch - menu 26

Under the standby mode, press button [menu] + [2] [6] he screen will display: SECOND

Press [menu] to enter, press ▲▼ to select ON/OFF and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - The use of stopwatch

When the stopwatch function is on, press [#lock] the stopwatch begins to count, press any key (except [#lock]) to stop the counting; press [#lock] to recount.

Channel name edition - menu 27

The channel name edition function can only be enabled under the channel mode rather than frequency mode.

Under the standby mode, press button [menu] + [2] [7] he screen will display: CHNAME

Press [menu] to enter and the cursor begins to flash (means this position is being edited). Press ▲▼ to select the demanded string and press A/B to continue the edition. Then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - The length of channel name is 8 strings at most. The channel is NO-NAME when all the 8 strings are null. The channel name can be four Chinese characters by frequency-writing software.

Channel storage - menu 28

Under the standby mode, press button [menu] + [2] [8] the screen will display: MEM-CH

Press [menu] to enter, press ▲▼ to select the demanded channel number and then press [menu] to confirm. Press [exit] to return to standby mode. For example, to store the receiving frequency 450.025MHz, receiving analog sub-audio 67.0Hz, and transmitting frequency 460.025 MHz to channel 10.

1. Input 450.025 frequency point under the frequency mode, press [menu] + [1] [5] to enter the receiving sub-audio function, press ▲▼ to select 67.0HZ, and [menu] to confirm.

2. Press button [menu] + [2] [4] to set the frequency difference as 10.000MHz and the frequency difference direction as positive.

3. Press button [menu] + [2] [8] to enter the storage channel, select channel 10 and press [menu] to confirm and return to the standby mode.

Attention - Under the standby mode, press [menu] + [2] [8] to enter the channel storage function, input the channel number and press [menu] to confirm. When the selected channel is null, the channel number will be displayed in blue. If the selected channel is the previously set channel, the channel number will be displayed in dark red.

Channel deletion - menu 29

Under the standby mode, press button [menu] + [2] [9] the screen will display: DEL-CH

Press [menu] to enter, press ▲▼ to select the channel number that needs to be deleted or input the channel number directly and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - The first channel cannot be deleted. When the selected channel is null, the channel number will be displayed in blue. If the selected channel is the previously set channel, the channel number will be displayed in dark red.

Sub-audio scanning - menu 30

Under the standby mode, press button [menu] + [3] [0] the screen will display: SCNCD

Press [menu] to enter the CTCSS scanning, which are analog sub-audio and digital sub-audio. When sub-audio frequency is detected, the detected sub-audio frequency will remain on the screen which can be saved by pressing [menu]. If the detected sub-audio is not the one required press [*scan] to continue the scanning.

Attention - When the current channel or frequency does not receive the corresponding carrier signal, scanning cannot be started.

Startup picture selection - menu 31

Under the standby mode, press button [menu] + [3] [1] the screen will display: PONMSG

Press [menu] to enter, press ▲▼ to select the required parameter and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - Two display modes: picture and batter level.

Silencing mode setting - menu 32

Under the standby mode, press button [menu] + [3] [2] the screen will display: SP-MUTE

Press [menu] to enter, press ▲▼ to select the required silencing mode and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - Silencing mode is the precondition of turning on the speaker. This setting is especially suitable for selected calling and group calling. This interphone has the following silencing modes:

• QT: when set in this mode, the speaker will be turned on by the matched carrier of sub-audio

• QT +DTMF: when set in this mode, only the matched carrier signal of DTMF which meets the QT conditions can tum on the speaker.

• QT/DTMF: when set in this mode, carrier signal that meets any of the QT condition or DTMF can turn on the speaker.

ID transmitting switch - menu 33

Under the standby mode, press button [menu] + [3] [3] the screen will display: ANI-SW

Press [menu] to enter, press ▲▼ to select ON/OFF and then press [menu] to confirm. Press [exit] to return to standby mode.

ID edition - menu 34

Under the standby mode, press button [menu] + [3] [4] the screen will display: ANI-EDIT

Press [menu] to enter input the demanded digits and then press [menu] to confirm. Press [exit] to return to standby mode.

The ID is made up of digits 0 - 9 and the first digit cannot be 0. The length is 3 digits at least and 6 digits at most.

Example 1: editing a 6-digit ID (123456)

Under the standby mode, press button [menu] + [3] [4] the screen will display: ANI-EDIT

Press [menu] and the first digit will flash. Input [1] [2] [3] [4] [5] [6] and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - Each interphone has only one ID. Areas A and B share the same ID.

Side-audio setting - menu 35

Under the standby mode, press button [menu] + [3] [5] the screen will display: DTMF-ST

Press [menu] to enter, press ▲▼ to select the demanded side-audio and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - ID code side-audio includes:

• DT-ST: the side-audio is on during transmission

• ANl-ST: ID code aide-audio is on during transmission

• DT=ANI: both side-audio and ID code aide-audio are on during transmission

• OFF: both are off.

Automatic keyboard lock- menu 36

Under the standby mode, press button [menu] + [3] [6] the screen will display: AUTOLOCK

Press [menu] to enter, press ▲▼ to select ON/OFF and then press [menu] to confirm. Press [exit] to return to standby mode.

When the automatic keyboard lock is ON, the keyboard will be locked if there is no operation in 15 seconds. Hold [#lock] for more than 2 seconds to unlock.

Attention - Manual lock: under tie standby mode, hold [#lock] for more than 2 seconds to lock the keyboard and hold [#lock] for more than 2 seconds to unlock.

Prior channel switch - menu 37

Under the standby mode, press button [menu] + [3] [7] the screen will display: PRICH-SW

Press [menu] to enter, press ▲▼ to select ON/OFF and then press [menu] to confirm. Press [exit] to return to standby mode.

For example, scan six normal channels CH1, CH2, CH3, CH4, CH5, and CH6, among which CH6 is the prior channel. Then the scanning process is CH1-CH6-CH2-CH6-CH3-CH6-CH4-CH6-CH5-CH6. If the interphone detects signal on prior channel, it will call out the frequency Prior channel can be selected by our frequency-writing software.

Channel scanning addition - menu 38

Other transition setting can be valid only when the transition function is on. Under the standby mode, press button [menu] + [3] [8] the screen will display: SCAN-ADD

Press [menu] to enter, press ▲▼ to select ON/OFF and then press [menu] to confirm. Press [exit] to return to standby mode.

Single-tone impulse frequency - menu 39

Under the standby mode, press button [menu] + [3] [9] the screen will display: ALERT

Press [menu] to enter, press ▲▼ to select demanded parameter and then press [menu] to confirm. Press [exit] to return to standby mode.

The frequencies are: 1000Hz, 1450Hz, 1750H, and 2100Hz.

Under the transmission mode, single-tone impulse frequency can be transmitted by pressing PF1.

Attention - In the relay system used for turning on single-tone impulse frequency, some relay needs to be initiated by a voice frequency single-impulse signal. If the relay has already been initiated, it is not needed to activate the impulse. The selectable impulse signals include 1750Hz, 2100HZ, 1000Hz, and 1450Hz.

ID transmission delay - menu 40

Under the standby mode, press button [menu] + [4] [0] the screen will display: PTT-DLY

Press [menu] to enter, press ▲▼ to select demanded time and then press [menu] to confirm. Press [exit] to return to standby mode.

The ID transmission delay is 100-3000 millisecond, which is divided into 30 levels. Each level has 100 milliseconds.

ID transmission mode - menu 41

Under the standby mode, press button [menu] + [4] [1] the screen will display: PTT-ID

Press [menu] to enter, press ▲▼ to select demanded mode and then press [menu] to confirm. Press [exit] to return to standby mode.

This interphone has 3 ID code transmission modes: BOT, EOT, and BOTH.

Ringing time - menu 42

Under the standby mode, press button [menu] + [4] [2] the screen will display: RING

Press [menu] to enter, press ▲▼ to select demanded time and then press [menu] to confirm. Press [exit] to return to standby mode.

The ringing time is 10 seconds, which is divided into 10 levels. OFF: turned off.

Channel scanning set A - menu 43

Scanning set means that the interphone divides the writing data channels into different groups and scans a certain group of channel by designating the group number.

Under the standby mode, press button [menu] + [4] [3] the screen will display: SCG-A

Press [menu] to enter, press ▲▼ to select and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - Channel scanning set A can only be realized in area A.

Channel scanning set B - menu 44

Scanning set means that the interphone divides the writing data channels into different groups and scans a certain group of channel by designating the group number interphone. Scanning set includes: all and 1-10 scanning unit.

Under the standby mode, press button [menu] + [4] [4] the screen will display: SCG-B

Press [menu] to enter, press ▲▼ to select and then press [menu] to confirm. Press [exit] to return to standby mode.

Attention - Channel scanning set B can only be realized in area B.

Transition receipt tone - menu 45

Under the standby mode, press button [menu] + [4] [5] the screen will display: RPT-TONE

Press [menu] to enter, press ▲▼ to select demanded parameter and then press [menu] to confirm. Press [exit] to return to standby mode.

ON: the receipt tone switch is on. OFF: the receipt tone switch is off.

Scanning sub-audio storage - menu 46

Under the standby mode, press button [menu] + [4] [6] the screen will display: SC-QT

Press [menu] to enter, press ▲▼ to select demanded parameter and then press [menu] to confirm. Press [exit] to return to standby mode.

1. Stored to transmission

2. Stored to receiving

3. All

Sub-frequency silencing setting - menu 47

Under the standby mode, press button [menu] + [4] [7] the screen will display: SMUTESET

Press [menu] to enter, press ▲▼ to select demanded parameter and then press [menu] to confirm. Press [exit] to return to standby mode.

OFF: turned off.

Attention

• TX: main frequency transmission, sub-frequency receiving volume off

• RX: main frequency receiving, sub-frequency receiving volume off

• TX/RX: main frequency transmission/receiving, main frequency transmission

Selected calling code selection - menu 48

Under the standby mode, press button [menu] + [4] [8] the screen will display: CALLCODE

Press [menu] to enter, press ▲▼ to select demanded selected calling code and then press [menu] to confirm. Press [exit] to return to standby mode.

There are 20 groups of selected calling code for selection. Selected calling code can be edited through frequency-writing software.

Background color - menu 49

Under the standby mode, press button [menu] + [4] [9] the screen will display: STYLES

Press [menu] to enter, press ▲▼ to select demanded color and then press [menu] to confirm. Press [exit] to return to standby mode. There are 6 background colors for selection: black, white, red, green, blue, and purple.

GPS - menu 50

Under the standby mode, press button [menu] + [5] [1] the screen will display: GPS

Press [menu] to enter, press ▲▼ to select ON/OFF and then press [menu] to confirm. Press [exit] to return to standby mode.

ADJ-TIME - menu 51

Under the standby mode, press button [menu] + [5] [0] the screen will display: ADJ-TIME

Press [menu] to enter, press ▲▼ to select -99 - +99 and then press [menu] to confirm. Press [exit] to return to standby mode.

Initialization setting - menu 52

Under the standby mode, press button [menu] + [5] [2] the screen will display: RESET

Initialization function parameter (VFO): initialize all functions and recover the factory functions. However, the channel parameter cannot be initialized. Initialization function parameter (ALL): initialize all functions and the channel parameters and recover the factory functions. Under the standby mode, press button [menu] + [5] [2] the screen will display: RESET. Press [menu] to enter, press ▲▼ to select demanded parameter, the screen will display: PLEASE WAIT…, which are [menu] initialization and all initialization.

Detailed explanation of certain functions

Frequency sweeping function

Detailed operation: under the standby mode, press [menu] + [*scan] function, the current frequency can be demodulated when the signal is received. If the current frequency is attached with a sub-audio, the sub-audio will show on the screen.

Group calling, selected calling, and unit calling

This interphone can transmit personal ID code, edit personal ID code and DTMF decode. It can also realize group calling, selected calling and unit calling without using other communication equipment. Preparatory work of using group calling, selected calling, and unit calling: ID code edition

Personal ID code: ID - XXX (3 digits) ID - XXXX (4 digits) ID - XXXXX (5 digits) ID - XXXXXX (6 digits)

Personal ID code mark+ group mark (1 - 9 group, 9 groups at most) + the only ID code of the group (00000 - 99999, 1000000 pieces at most).

Note: the ID code of interphone inside the group must be unique.

Detailed processes are as follows:

a. Using group calling function

Press PTT to transmit and press [*scan] + [#lock] when the transmission is completed.

b. Using unit calling function

Press PTT to transmit and input group number (first digit of the

receiving user's ID code+[*scan] + [#lock]) when the transmission is completed.

c. Using selected calling function

Press PTT to transmit and input the personal ID code when the

transmission is completed.

Selected calling key can be used for directly calling the selected interphone through the software we provide. Set the selected calling code the same as the personal ID code of the receiving interphone. Dress selected calling key and the interphone will automatically send the ID code of your interphone + ID code of receiving interphone. For example, the ID code of receiving interphone is 123466 and the

Interphone selected calling number is also 123456.

Troubleshooting

| Failure | Solution |
| --- | --- |
| Continuous receiving indicator but the speaker makes no sound | • Make sure that volume button is set to the maximum value • Check the setting of sub-audio • Check the silencing mode |
| No response when pressing a key | • Whether the keyboard is locked • Whether other keys have been stuck |
| Unable to scan | • Check the channel for scanning groups |
| Automatic transmission under standby mode without pressing PPT | • Check whether the voice-control function is on and whether the level is too low |
| Other sound in the channel (not from the members) | • Please change the frequency of sub-audio voice |